# 5     Notification Configuration

This chapter discusses how to configure event notifications.  Topics include the following:

- Description of the notification function

- Configuring notification options

- Configuring a modem

- Configuring fax notification

- Configuring pager notification

- Configuring broadcast notification

- Configuring e-mail notification

- SNMP Trap notification

###### Notification Processing Center

- A management program is much more useful when it includes a reporting or notification function.  For this reason, RAIDWatch allows you to configure notifications which occur in response to various disk array events.  The notification application is called, “Notification Processing Center” or NPC.

The notification function makes it possible to manage (or monitor) the disk array system even when the administrator is away from the manager station.  When properly configured, this function can notify the administrator of event occurrences in a RAID array instantly even when he or she is at home, or in other places around the globe.

RAIDWatch NPC provides five methods of sending notifications:

- Fax

- Pager

- E-mail

- Broadcast

- SNMP Traps

You can use any or all of the notification methods mentioned above.  In order to use a particular method, you must have the proper hardware (like a modem for pager notification) and software (like Windows NT with the window messaging subsystem for e-mail message notification) installed in your system.

## 5.2     Before You Begin

- Before RAIDWatch’s event notification can be used, some adjustments may need to be made to your OS’s configuration.  See 2.3 Platform Requirements for information about pre-installation configuration needs.  Many of the steps described in section 2.3 directly relate to event notification, particularly for Windows OSes.

IMPORTANT:  All configuration entries in all notification types will send event notifications at every event.  Multiple receiver destinations are available in all types, but every event notification will be sent to all configured receivers in all types.  That is, over-configuring may result in an unintentional “storm” of event notifications.

## 5.3     Basic Running Requirements

- Notification Processing Center (NPC) is a separate software application from RAIDWatch primary and secondary agents, and from RAIDWatch Manager, and it must be manually configured and run (and re-run after a host computer reset) in order for notifications to be sent out.

Installation and configuration of the NPC is accomplished using the same program and interface as that for the installation and configuration of the Primary and Secondary agents.

In order for NPC to work, it must be installed and run on the same host computer as a Primary Agent.  Once NPC is installed, it must be configured to send the types of notifications you want to receive.

- IMPORTANT NOTE:

For NPC to function on NT servers, the "Windows Messaging Service," "Personal Fax," "Internet Mail," and "Telephony service" must be installed and running.  Windows Messaging can be used to receive and store faxes, and for other kinds of messages.

## 5. 4     Redundant NPC Modules

NPC and Primary Agent should be installed redundantly on two different servers in case the Primary Agent or NPC module should fail.  If the Primary Agent or NPC should fail, the Primary Agent and NPC installed on another server will continue event notification.  This method is applied for the consideration that fatal system failure might occur unnoticed at the time when the only NPC or Primary Agent is unable to function.

Redundant NPC modules should not be installed without Primary Agents.  Run the RAIDWatch configuration procedure and enable this function by designating another server as where the standby modules will be installed.  Primary Agent and NPC module must then be installed manually on that particular server (either a RAID server or not).  The installation utility does not automatically add these modules to the selected server.

## 5.5     Severity Levels

- RAIDWatch classifies disk array events into three severity levels.  The first level includes non-critical information events such as initialization of the controller and initiation of the rebuilding process.  Level 2 severity includes events which require the issuance of a warning message.  Level 3 severity is the most serious level, and includes events that need immediate attention.  The following provides example events for each level:

Level 1 Severity Events (examples)

- Controller Initialization Complete

- Rebuild Initiated

- Rebuild Complete

- Rebuild Failed

- Logical Drive Initialization Started

- Logical Drive Initialization Complete

- Logical Drive Initialization Failed

- Level 2 Severity Events (examples)

- SCSI Target Select Timeout

- SCSI Target Phase/Signal Error

- SCSI Target Unexpected Disconnect

- SCSI Target Negotiation Error

- SCSI Target Timed Out

- SCSI Target Parity Error

- SCSI Bus Reset Issued

- SCSI Target Not Ready Error

- SCSI Target Media Error

- SCSI Target Hardware Error

- SCSI Target Unit Attention

- SCSI Target Unexpected Sense Date

- SCSI Target Block Reassignment Failure

- SCSI Target Unexpected Data Over/Underrun

- Drive SCSI Unrecognized Event

- Level 3 Severity Events (example)

- SCSI Drive Failure

- NOTE: The current version of NPC does not include event notification configuration based on event severity.  Future versions of NPC may support event severity based notifications.

## 5.6      Configuring Notification Options

- In order to configure NPC notification modes, you must run the RAIDWatch installation procedure for your particular OS and arrangement (e.g., local or remote).  Once you start the installation procedure (see chapter 2 for information on how to do this), choose “Configure Only” and then double-click “Notification” in the upper left part of the screen.  NPC configuration options should be listed and include:

- Modem

- Fax

- Pager

- Broadcast

- E-Mail

- SNMP

From the list of notification method options, choose the one you would like to configure and click on it.

## 5.7     Configuring Modem Settings

- NPC needs to be configured with the proper settings for the host’s fax/modem if fax or pager notification is going to be used.

NPC modem settings include Serial Port and Baud Rate.  If multiple modems are available on different COM ports, each one can be entered.  The NPC settings of each of these variables must identically match the host’s fax/modem configuration settings.

- Serial Port – choose from COM1 to COM16 in the drop box.

- Baud Rate – choose 4800, 9600, 19200, 38400, or 57600 in the drop box.

When you have finished, click Add to allow NPC to use this modem.  Next, choose another NPC item to configure or click Close to save changes and exit the installation procedure.

- Important Note:

- A pre-configured modem can only be removed from the list of Available Modems after all references to the modem have been removed.

## 5.8     Configuring Fax Notification

- NPC sends fax messages to specific destinations using the fax service and window messaging subsystem.  If these services are not available on your host computer, install them first before using the fax notification function of the system.  You will also need to connect a fax/modem to your computer.  Installing a fax/modem can be a tricky process, so consult your operating system manuals for instructions on how to install your fax/modem software.

As a rule, the following services should be available on the host computer running NPC:

- Personal Fax

- Windows Messaging

- Internet Mail and Telephony Services

For fax notification to function on a Unix-based system, connect a modem and NPC will be able to send fax.

To enable fax service on NT, the following steps can be taken as examples:

- You need to install Personal Fax and Windows Messaging.  Personal Fax is usually bundled with Microsoft's Small Business Server.  You may download its package from Microsoft's web site.

- Here are some of the known issues with installing Personal Fax.

- Telephony service must be enabled and running.

- Listed below are examples for configuring fax service under Windows NT.  Some of them must be configured during the installation process of Personal Fax.

- When Personal Fax is installed, only administrators can send faxes.  NPC has altered the associated configurations to eliminate this limitation. However, it is recommended to log in as an "Administrator."  For normal users to send faxes, you may check your write access in the registry key:

HKEY_LOCAL_MACHINE/SYSTEM/CurrentControlSet/Control/Print/Printers/Fax/.…

- The profile name necessary for logging on to personal fax/messaging service is usually the same as the account name that you use for system logon.

Fax service requires Windows Messaging to send fax via email.

- You may add more recipient machines later using the RAIDWatch configuration utility (as will be discussed in the proceeding discussions).

- Proceed to complete the installation process.

- Diagrammed below is the topological view of the relationship between fax receiving machines and the fax sending computer.

- Add Fax Mail Transport and Internet Mail as the active services to Windows Messaging profile.

Since NPC depends largely on the fax service installed to the MAPI of Windows NT for the delivery of fax messages, most fax notification parameters are configured through this software package.  NPC allows you to set a telephone number for the receiving fax machine, the message that will be sent in the fax, the modem that will be used from a list of available modems, and the initialization string the NPC will use when starting to send a fax.

- Profile Name –  is a collection name required for logon to a Windows Messaging profile (FAX, E-mail, Exchange Mail).  It is usually the same with the account name.  A profile name validates the available messaging services and service providers during a particular MAPI session.

- Telephone – enter the phone number of the event notification receiving fax machine.  Note that you must enter the entire dialing sequence (no hyphens) including line access, country and area codes.

- Message – the message that will be printed on the fax message.  You may add, for instance, the contact information of technical personnel, etc.

- Available Modems – choose a modem from those listed in the drop box.

- Initializing String – is a standard AT command set initialization command string.  The default is &F1X0M0 and should be appropriate for most modems.  If you need to change it, consult your modem manual for the correct AT command string.

The Initializing String is applied for fax and pager service on Unix-based systems.

When you have finished, click Add to have NPC send event notifications to this fax machine.  Next, either enter another fax machine to receive notices, choose another NPC item to configure, or click Close to save changes and exit the installation procedure.

## 5.9     Configuring Pager Notification

- Pager notifications, like faxes, are sent using the fax/modem, so an NPC modem configuration is required.

NPC allows you to set a telephone number for the receiving pager, the message that will be sent to the pager, the modem that will be used from a list of available modems, and the initialization string the NPC will use when starting to send a pager message.

- Telephone – enter the phone number of the event notification receiving pager.  Note that you must enter the entire dialing sequence (no hyphens) including line access, country and area codes.

- Message – the message that will be delivered to the pager.

- Available Modems – choose a modem from those listed in the drop box.

- Initializing String – is a standard AT command set initialization command string for Unix-based systems. The default is V1B1E0F0L1M1Q0TV1X1Y0&C1&D2&G0S7=20 and should be appropriate for most modems sending to a pager.  If you need to change it, consult your modem manual for the correct AT command string.

When you have finished, click Add to have NPC send event notifications to this pager.  Next, either enter another pager to receive notices, choose another NPC item to configure, or click Close to save changes and exit the installation procedure.

## 5.10    Configuring Broadcast Message Notification

- Broadcast messages are sent to every station on the subnetwork configured.  A broadcast message should cause a window to appear that will stay visible until manually closed by a user.

To enable Message Broadcast in Solaris, open a console terminal and then execute "./rpc.rwalld" under "/usr/lib/netsvc/rwall".

Make sure NETBIOS and TCP/IP are active services on your NT for message broadcasting.

Broadcast settings include:

- Host Name – which should be computer name for Windows NT/98/95 but should be a host name or IP address for Unix systems such as Solaris, Linux, HP-UX, and AIX.

- Message – which is the message that network stations will see when events occur.  Message added here can serve as a reminder and may as well be the technical contact information.

NOTE:  Cross-platform broadcasts (i.e., Unix-based servers broadcasting to Windows-based network stations and vice versa) are not currently supported.

When you have finished, click Add to have NPC send event notification broadcasts to stations served by this server.  Next, either enter another server to receive notices, choose another NPC item to configure, or click Close to save changes and exit the installation procedure.

## 5.11    Configuring E-Mail Notification

- NPC sends e-mail messages to specific people on the network or Internet using the Microsoft Internet Mail service or a third-party, MAPI-compliant e-mail service.  You will need these services to enable the e-mail notification function of NPC.

Since NPC depends largely on services mentioned above for the delivery of e-mail messages, most e-mail notification parameters are configured through these packages.  NPC only allows you to select the users who will receive the notification messages, and specify the notification message for each recipient.

For information on how to configure users for e-mail reception and other pertinent information about the Microsoft Internet Mail service in Windows NT, refer to the Windows NT manual.  Be sure that these services are configured properly, and functioning before using the e-mail notification function of NPC.

NPC e-mail settings include:

- Profile Name –  is a collection name required for logon to a Windows Messaging profile (FAX, E-mail, Exchange Mail). It is usually the same with the account name.  A profile name validates the available messaging services and service providers during a particular MAPI session.

- SMTP Server – enter the IP address of the mail server to use to send e-mail event notifications.

- Sender’s E-mail – enter the valid e-mail address that will be used as the “From” part of event e-mail notices.

- Receiver’s E-mail – enter the full internet e-mail address for the receiver.

- Subject – enter a subject for notification e-mails.  Currently, Subject is not available for Unix-based platforms.

- Message – enter the message to be included as the  e-mail event notices.  Messages can be added such as the contact information of technical personnel.

When you have finished, click Add to have NPC send event notification e-mail messages to this e-mail address.  Next, either enter another e-mail address to receive notices, choose another NPC item to configure, or click Close to save changes and exit the installation procedure.

## 5.12    Configuring SNMP Trap Notification

- Starting SNMP trap receiving capability can be as simple as keying "net start snmp" and "snmputil trap" at the DOS prompt under Windows-based systems.

To enable SNMP receiving capability under Linux, you can use the "snmptrapd -P" command.

SNMP trap notification is a supported NPC function.  Simply enter the IP addresses of SNMP agents for trap notification to work.

Avail hosts to be listening to SNMP traps by entering its IP addresses and adding the addresses to the list of trap receivers.

Users may select a severity parameter (1,2, or 3).  The parameter determines events of what severity level(s) are to be sent via SNMP.

| 1 | All severity levels |
| --- | --- |
| 2 | Two levels: Warning and Alert |
| 3 | Only the most serious events: Alert messages |