[ PROTOCOLS:MSTD-1777-TESTS1.DOC ]                             [ 4/88 ]

DCEC PROTOCOL LABORATORY

INTERNET PROTOCOL

MIL-STD-1777

CERTIFICATION TESTS INDEX

This Certification Tests Index provides information on the derivation,

organization, and function of tests specified for Internet Protocol

(IP) within the DCEC Protocol Laboratory.

This document is divided into five sections:

IP TRACEABILITY INDEX;

IP TESTS INDEX;

IP CODES INDEX;

IP TEST SCENARIOS INDEX;

and IP SCENARIOS AND TESTS DESCRIPTIONS.

--------

IP TRACEABILITY INDEX:  IP TEST NUMBERS versus IP MIL-STD-1777

References...

The table shows,  for MIL-STD-1777,  the IP Test Numbers which may be

regarded as the "principal  tests" of each required function,

operation, option, mode, response, or state.

--------

IP TESTS INDEX:   IP TEST NUMBERS versus IP Commands/Primitives/

Options/Modes...

The table shows the IP Test Numbers which may be regarded as the

"principal tests" of each IP Command or Primitive and/or Option or

Mode.

--------

IP CODES INDEX:   IP TEST NUMBERS versus IP Response Codes...

The table shows the IP  Test Numbers which may be regarded as the

"principal  tests" of each IP Response (or State, etc.) Code to/for/of

the indicated IP Commands or Primitives.

--------

IP TEST SCENARIOS INDEX:    IP TEST SCENARIO FILES versus IP TEST

NUMBERS...

The table shows, for each IP Test Number, the UNIX filenames of the I

Test Scenario Files in which it may be regarded as a "principal test

objective".

--------

IP SCENARIOS AND  TESTS DESCRIPTIONS...

This section provides a brief narrative of the scope and objectives of

each IP Test Scenario File and a narrative or graphic operational

description of each IP Test Number.

IP TRACEABILITY INDEX

IP TEST NUMBERS

versus

IP MIL-STD-1777 References

The following table shows, for MIL-STD-1777, the IP Test Numbers which

may be regarded as the "principal tests" of each required function,

operation, option, mode, response, or state.  Test Numbers not

indicated here may be used to establish necessary predecessor

conditions for these tests.

Reference                                     Test Number

---------                                     ----------------------

9.1                                           570-573, 2000-2099,

2100-2142, 2200-2205,

2904-2913

9.2.1                                         1100, 1108, 1113

9.2.1.1                                       801

9.2.1.1.1                                     801

9.2.1.1.2                                     1102

9.2.1.2                                       1101

9.2.1.2.1                                     1100, 1105, 1108,

1113, 2100-2142, 3102

9.2.2                                         68, 500-525, 1024,

1144

9.2.2.1                                       68, 1113, 1322

9.2.2.2                                       500-509, 517-525, 576

9.2.2.3                                       510-512

9.2.3                                         800-805

9.2.4                                         600-617

9.2.5                                         200-223, 300-315,

400-410

9.2.6                                         1000-1047, 3000-3046

9.2.6.1                                       1113

9.3                                           804

Reference                                      Test Number

-----------                                    ----------------

9.3.1                                          100, 101, 111

9.3.2                                          102-104, 111

9.3.3                                          200-223, 300-315

9.3.4                                          105-107, 111

9.3.5                                          0, 21845, 43690,

65535

9.3.6                                          500, 570, 571

9.3.7                                          525, 573

9.3.8                                          600-617

9.3.9                                          801, 2903

9.3.10                                         800-805

9.3.11                                         801

9.3.12                                         801

9.3.13                                         1000-1047,  1101, 1109,

1113, 1301, 1309,

2000-2011, 3000-3046,

4000-4011

9.3.13.1                                       1000-1047, 1101, 1109,

1113, 1301, 1309,

3000-3046

9.3.14                                         1000, 3000

9.3.15                                         2000-2099, 2200-2205,

4000-4099, 4200-4205

9.3.15.1                                       1322, 3122

9.3.15.2                                       1001-1004, 3001-3004

9.3.15.3                                       1005-1012,  1322,

2013-2026, 3005-3012,

4013-4026

9.3.15.3.1                                     1005-1012, 3005-3012

9.3.15.3.2                                     578, 579

Reference                                      Test Numbers

---------                                      ----------------------

9.3.15.3.3                                     580, 581

9.3.15.3.4                                     582, 583, 584

9.3.15.4                                       1101, 1102, 1105,

1300, 2100-2119, 3100,

3101, 3102, 4100-4119

9.3.15.5                                       1109, 1111, 1112,

1308, 2120-2139, 3108,

3109, 4120-4139

9.3.15.6                                       1113, 1315, 2043-2062,

3122, 4043-4062

9.3.15.7                                       1013-1030, 2028-2039,

3013-3030, 4028-4039

9.3.15.8                                       1036-1047, 2063-2099,

3036-3046, 4063-4099

9.4.6.1.1                                      68, 801, 1144,

4000-4099, 4100-4142

9.4.6.1.2                                      100-111, 500, 600-608,

900, 2901-2903

9.4.6.1.3                                      500, 2970, 2972

9.4.6.1.4                                      510-512, 515-519

9.4.6.2.1                                      500

9.4.6.2.2                                      68, 1144

9.4.6.2.3                                      800-805

9.4.6.2.4                                      2902, 2903

9.4.6.2.5                                      1024, 1144

9.4.6.2.6                                      500-525

9.4.6.2.7                                      100-111

9.4.6.2.8                                      600-607

9.4.6.2.9                                      608-615, 4000-4099,

4100-4142, 4200-4205

9.4.6.2.10                                     801, 1100

Reference                                     Test Numbers

-----------                                   ------------------------

9.4.6.2.11                                    800, 1100, 1300, 2903

9.4.6.3.1                                     2900-2912, 2930-2934,

2950-2969

9.4.6.3.2                                     68, 208-215, 308-315,

801, 804, 3100

9.4.6.3.3                                     801, 804

9.4.6.3.4                                     2903

9.4.6.3.5                                     100-110, 512,

2200-2205

9.4.6.3.6                                     2904-2912

9.4.6.3.7                                     1024, 1144

9.4.6.3.9                                     500-525, 574-585

9.4.6.3.10                                    500, 525

9.4.6.3.11                                    510-512, 515-519

9.4.6.3.12                                    801

9.4.6.3.13                                    1100-1121, 1300-1322,

3100-3122

IP TESTS INDEX

IP TEST NUMBERS

versus

IP Commands/Primitives/Options/Modes

The following table shows the IP Test Numbers which may be

regarded as the "principal tests" of each IP Command or Primitive

and/or Option or Mode.  Test Numbers not indicated here may be

used to establish necessary predecessor conditions for these

tests.

Command or Primitive      Option or Mode      Test Number

--------------------      --------------      ------------------

VERSION NUMBER            ==>                 100-101, 801

too small           100

too large           101

correct             801

INTERNET HEADER LENGTH    ==>                 102-104, 801

too large           102

too small           103

zero                104

correct             801

TYPE OF SERVICE           ==>                 200s, 300s, 400s

Gateway Testing           ==>                 400s

low delay

high reliability    400

low delay

high throughput     401

high reliability

maximum precedence  402

low delay

high reliability

maximum precedence  403

high throughput

maximum precedence  404

high throughput

Command or Primitive     Option or Mode      Test Number

-------------------      --------------      -----------------

high reliability

maximum precedence  405

low delay

low precedence      406

low delay

high reliability

low precedence      407

low delay

high throughput

high reliability

very low precedence 408

low delay

high throughput

high reliability

high precedence     409

low delay

high throughput

high reliability

maximum precedence  410

Precedence               ==>                 200s

bottom-up (received

by IUT)    ==>                 200-207

7                   200

6                   201

5                   202

4                   203

3                   204

2                   205

1                   206

0                   207

top-down (sent by IUT) ==>                 208-215

7                   208

6                   209

Command or Primitive     Option or Mode      Test Number

--------------------     --------------      -----------------

5                   210

4                   211

3                   212

2                   213

1                   214

0                   215

effect                 ==>                 216

Delay

Throughput

Reliability              ==>                 300s

bottom-up (received

by IUT)    =                   300-307

delay = 0

throughput = 0

reliability = 0     300

delay = 0

throughput = 0

reliability = 1     301

delay = 0

throughput = 1

reliability = 0     302

delay = 1

throughput = 0

reliability = 0     303

delay = 0

throughput = 1

reliability = 1     304

delay = 1

throughput = 0

reliability = 1     305

delay = 1

throughput = 1

reliability = 0     306

Command or Primitive      Option or Mode      Test Number

--------------------      --------------      ------------------

delay = 1

throughput = 1

reliability = 1     307

top-down (sent by IUT)  ==>                 308-315

delay = 0

throughput = 0

reliability = 0     315

delay = 0

throughput = 0

reliability = 1     314

delay = 0

throughput = 1

reliability = 0     313

delay = 1

throughput = 0

reliability = 0     312

delay = 0

throughput = 1

reliability = 1     311

delay = 1

throughput = 0

reliability = 1     310

delay = 1

throughput = 1

reliability = 0     309

delay = 1

throughput = 1

reliability = 1     308

TOTAL LENGTH              ==>                 105-107  801

too large           105

too small           106

excessively large   107

correct             801

IDENTIFICATION            ==>                 0,21845,43690,65535

bits clear each way 0

Command or Primitive      Option or Mode      Test Number

--------------------      --------------      -------------------

bits set each way   65535

odd bits

set each way        43690

even bits

set each way        21845

bits clear to IUT

and set to Tester   65535

bits set to IUT

and clear to Tester 0

odd bits set to IUT

and even bits set

to Tester           21845

even bits set to IUT

and odd bits set

to Tester           43690

FLAGS & FRAGMENT OFFSET   ==>                 500s, 2970s

Normal                    ==>                 500-525, 2970, 2971

2 fragments         500

3 fragments         501

4 fragments         502

4 fragments where

second and third

are out of order    503

4 fragments in

reverse order       504

4 fragments that

are delayed within

relatively large

time to live limits 505

4 fragments that

are delayed within

relatively small

time to live limits 506

duplicate

header fragment     507

Command or Primitive      Option or Mode      Test Number

--------------------      --------------      --------------------

duplicate

middle fragment     508

duplicate

tail fragment       509

missing

header fragment     510

missing

middle fragment     511

missing

tail fragment       512

simultaneous

reassembly          513-514

attempt

simultaneous

reassembly with

a missing fragment  515-516

indentification

separation          517

protocol

separation          518

middle fragment     519

reassemble a

576 byte datagram

split on 8 octet

boundaries          520

reassemble a

576 byte datagram

split on 16 octet

boundaries          521

reassemble a

576 byte datagram

split on 32 octet

boundaries          522

reassemble a

576 byte datagram

split on 64 octet

boundaries          523

Command or Primitive      Option or Mode      Test Number

--------------------      --------------      ------------------

reassemble a

576 byte datagram

split on 128 octet

boundaries          524

reassemble a

576 byte datagram

split on 256 octet

boundaries          525

ICMP                2970

ICMP reverse order  2971

Abnormal                  ==>                 570-585, 2972, 2973

MF and DF set       570

DF set and

fragment offset

not 0               571

enormous

fragment offsets    572

range of

fragment offsets    573

precedence

mismatch            574

security level

mismatch with

unclassified

exception           575

classified security

level mismatch      576

security level

mismatch with

classified

exception           577

first

compartment byte

mismatch            578

second

compartment byte

mismatch            579

Command or Primitive      Option or Mode      Test Number

--------------------      --------------      ------------------

first

handling byte

mismatch            580

second

handling byte

mismatch            581

first

transmission

control code byte

mismatch            582

second

transmission

control code byte

mismatch            583

third

transmission

control code byte

mismatch            584

security option

present exception   585

ICMP with bogus

ICMP fragments      2972

ICMP with a

fragment with a

time to live of 0   2973

TIME TO LIVE              ==>                 600s

bottom-up (received

by IUT)      ==>                 600-608

0                   607

1                   608

20                  604

85 (odd bits set)   605

127                 603

128                 602

129                 601

Command or Primitive      Option or Mode      Test Number

--------------------      --------------      ------------------

170 (even bits set) 606

255                 600

top-down (sent by IUT)    ==>                 609-617

0                   617

1                   616

20                  613

85 (odd bits set)   614

127                 612

128                 611

129                 610

170 (even bits set) 615

255                 609

PROTOCOL                  ==>                 574, 801, 2900s

Reassembly Mismatch       ==>                 574

Remote Driver             ==>                 801

Internet Control

Message Protocol          ==>                 2900s

ICMP Echo Request       ==>                 2901-2903

bad header checksum 2901

bad ICMP checksum   2902

both checksums good 2903

fragmented          2970, 2971

ICMP Parameter Problem ==>                  2904

ICMP Redirect           ==>                 2950-2969

primer datagram

for host redirects 2950

Command or Primitive      Option or Mode      Test Number

--------------------      --------------      ------------------

positive

host redirect       2951-2952

negative host and

type of service

redirect            2953-2954

host redirect and

host and type of

service redirect

independence        2955

negative

host redirect       2956, 2958

positive host and

type of service

redirect            2957, 2959

primer datagram for

network redirects   2960

positive

network redirect    2961-2962

negative network and

type of service

redirect            2963-2964

network redirect and

network and type of

service redirect

independence        2965

negative

network redirect    2966, 2968

positive network and

type of service

redirect            2967, 2969

ICMP Source Quench      ==>                 2930-2934

primer datagram to

be sent by IUT so

it can be told to

quiet down          2930

Source Quenchs      2931-2934

ICMP Timeout            ==>                 2905-2906

Command or Primitive      Option or Mode      Test Number

--------------------      --------------      ------------------

time to live        2905

reassembly          2906

ICMP Unreachable        ==>                 2907-2913

network             2907

host                2908

protocol            2909

port                2910

fragmentation       2911

source route        2912

send a normal IP

datagram for (are

you there?) check   2913

HEADER CHECKSUM           ==>                 800-805

too small           800

correct             801

too large           802

too small for IP

header with option  803

correct for IP

header with option  804

too large for IP

header with option  805

SOURCE ADDRESS            ==>                 801

DESTINATION ADDRESS       ==>                 801, 900, 2952

OPTIONS                   ==>                 1000-1047, 1100-1121,

1300-1323, 2000-2099,

2100-2142, 2200-2205,

3000-3046, 3100-3122,

4000-4099, 4100-4142,

4200-4205

Bad Routing               ==>                 2100-2142, 4100-4142

Command or Primitive      Option or Mode      Test Number

--------------------      --------------      ------------------

bottom-up              ==>                 2100-2142

top-down               ==>                 4100-4142

Gateway Routing           ==>                 1100-1121

Host Routing              ==>                 1300-1323, 3100-3122

bottom-up              ==>                 1300-1323

top-down               ==>                 3100-3122

Missing Option Length     ==>                 2200-2205, 4200-4205

bottom-up              ==>                 2200-2205

top-down               ==>                 4200-4205

Non-Routing               ==>                 1000-1047, 2000-2099,

3000-3046, 4000-4099

bottom-up               ==>                 1000-1047, 2000-2099

Bad                    ==>                 2000-2099

Good                   ==>                 1000-1047

top-down                ==>                 3000-3046, 4000-4099

Bad                    ==>                 4000-4099

Good                   ==>                 3000-3046

DATA                      ==>                 68, 576, 1024,  1144

send a 68

byte datagram       68

receive a 576

byte datagram       576

send a 1024

byte datagram       1024

send an 1144

byte datagram       1144

IP CODES INDEX

IP TEST NUMBERS

versus

IP Response Codes

The following table shows the IP Test Numbers which may be

regarded as the "principal tests" of each IP Response (or State,

etc.) Code to/for/of the indicated IP Commands or Primitives.

Test Numbers not indicated here may be used to establish

necessary predecessor conditions for these tests.

Response Code       Command or Primitive      Test Number

--------------      --------------------      ------------------

datagram            datagram                  801

datagram with

end of option

list option         datagram                  1000

datagram with

no operation

option              datagram                  1001

datagram with

security

option              datagram                  1005

datagram with

loose source and

record route

option              datagram                  1100, 3100

datagram with

strict source and

record route

option              datagram                  1108, 3108

datagram with

record route

option              datagram                  1113, 3122

datagram with

stream ID

option              datagram                  1029

datagram with

timestamp

option              datagram                  1113, 1117, 1119,

3036, 3040, 3044

fragmented

datagram            datagram                  1024, 1144

Response Code        Command or Primitive     Response Code

---------------      --------------------     ------------------

ICMP Destination

Unreachable

message           datagram                 900,  1105, 1112

ICMP Echo Reply      ICMP Echo Request

message               message              2903

ICMP Parameter

Problem

message           datagram                 100, 570, 2200

ICMP Time Exceeded

message           datagram                 511, 608

IP TEST SCENARIOS INDEX

IP TEST SCENARIO FILES

versus

IP TEST NUMBERS

Within the PROTOLAB VAX-11/750 Test Master System, all IP test

Scenario Files may be found in the following UNIX directory;;

/usr/users/TSL/test/ip

The following table shows, for each IP Test Number, the UNIX

filenames of the IP test Scenario Files in which it may be

regarded as a "principal test objective".   A Test Number may be

used, to establish necessary predecessor conditions for other

Test Numbers, in Scenario Files not indicated here.

Test Number               Scenario File

-----------------------   ---------------------------------------------

0                         IdentTest

68                        Send68Test

100-111                   BadHeaderTest

200-207                   PrecBotTest

208-215                   PrecTopTest

216-223                   PrecEffTest

300-307                   TOS_BitBotTest

308-315                   TOS_BitTopTest

400-410                   TOS_GateTest

500-525                   ReasMustTest

570-585                   OddReasTest

576                       Send576Test

576                       Whole576Test

600-606                   TTLGdBotTest

607-608                   TTLBdBotTest

609-615                   TTLGdTopTest

Test Number               Scenario File

-----------               -------------

616-617                   TTLBdTopTest

800-805                   HdrCksumTest

900                       BadDestTest

1000-1047                 OptGdBotTest

1024                      Send1024Test

1100                      OpGlSrcBotTest

1101                      OpG2SrcBotTest

1102                      OpG2SrcBotTest

1103                      OpG3SrcBotTest

1104                      OpG3SrcBotTest

1105                      OpG2SrcBotTest

1106                      OpG6SrcBotTest

1107                      OpG3SrcBotTest

1108                      OpGlSrcBotTest

1109                      OpG2SrcBotTest

1110                      OpG3SrcBotTest

1111                      OpG2SrcBotTest

1112                      OpG2SrcBotTest

1113                      OpG1SrcBotTest

1114                      OpG1SrcBotTest

1115                      OpG2SrcBotTest

1116                      OpG2SrcBotTest

1117                      OpG2SrcBotTest

1118                      OpG2SrcBotTest

1119                      OpG2SrcBotTest

Test Number               Scenario File

-----------               ---------------

1120                      OpG2SrcBotTest

1121                      OpG6SrcBotTest

1144                      Send1144Test

1300                      OpG1DstBotTest

1301                      OpG2DstBotTest

1303                      OpG3DstBotTest

1306                      OpG6DstBotTest

1307                      OpG3DstBotTest

1308                      OpG1DstBotTest

1309                      OpG2DstBotTest

1310                      OpG3DstBotTest

1311                      OpG2DstBotTest

1312                      OpG3DstBotTest

1313                      OpG1DstBotTest

1314                      OpG1DstBotTest

1315                      OpG2DstBotTest

1316                      OpG2DstBotTest

1317                      OpG2DstBotTest

1318                      OpG2DstBotTest

1319                      OpG2DstBotTest

1320                      OpG2DstBotTest

1321                      OpG2DstBotTest

1322                      OpG1DstBotTest

1323                      OpG2DstBotTest

2000-2099                 OptBdBotTest

Test Number               Scenario File

-----------               ---------------

2100-2142                 OpBdRouBotTest

2200-2205                 NoOpLenBotTest

2900-2913                 ICMPart1Test

2930-2934                 ICMPSrcQueTest

2950-2969                 ICMPRedrctTest

2970-2974                 ICMPReasTest

3000-3046                 OptGdTopTest

3100                      OpGlSrcTopTest

3101                      OpG2SrcTopTest

3102                      OpG2SrcTopTest

3103                      OpG3SrcTopTest

3106                      OpG6SrcTopTest

3108                      OpG1SrcTopTest

3109                      OpG2SrcTopTest

3110                      OpG3SrcTopTest

3113                      OpG1SrcTopTest

3114                      OpG1SrcTopTest

3115                      OpG2SrcTopTest

3116                      OpG2SrcTopTest

3117                      OpG2SrcTopTest

3118                      OpG2SrcTopTest

3119                      OpG2SrcTopTest

3120                      OpG2SrcTopTest

3122                      OpG1SrcTopTest

4000-4099                 OptBdTopTest

Test Number               Scenario File

-----------               ---------------

4100-4142                 OpBdRouTopTest

4200-4205                 NoOpLenTopTest

21845                     IdentTest

43690                     IdentTest

65535                     IdentTest

IP SCENARIOS AND TESTS DESCRIPTIONS

WARNING:  An implementation may only be denied certification if

it does not conform to MIL-STD-1777 or to the Internet Protocol

Remote Driver Specification. These specifications, interpreted as

required by law, are the final authorities.

While an effort has been made to identify every conceivable legal

response sanctified by the MIL-STD, implementors are invited to

explain their non-matching responses using the MIL-STD.

NOTICE: Many tests are intended to be controversial in order to

induce the Department of Defense to clarify MIL-STD-1777.

Scenario File BadDestTest: "Bad Destination Address" datagram

900       destination address doesn't match host's

------------------------------

Test 900 destination address doesn't match host's

-Can the IUT detect that datagram isn't meant for it?

-Action:  Send a datagram from the IP Tester to the host IP

Implementation Under Test whose destination address

indicates that it is for another host. The IUT should

discard the datagram. It may send an ICMP Unreachable Host

message to the IP Tester.

-Verification: If there is a response datagram displayed by

the IP Tester, its display should contain the string "ICMP

DESTINATION UNREACHABLE ID=00900 HOST". The string "ident =

900" should not appear in the display of the response

datagram because this would indicate that the datagram was

passed by the IUT to its Remote Driver.

Success: An Internet Control Message Protocol (ICMP)

Unreachable Most message displayed by the IP Tester. The

total lack of an IP Tester response datagram display is also

an acceptable response.

Failure: A response datagram whose IP Tester display

contains the string "ident = 900".

Scenario File BadHeaderTest: "Bad Header Value" datagrams

100       version number is 3

101       version number is 5

102       internet header length excessive

103       internet header length short

104       internet header length is 0

105       total length exceeds SNP length

106       total  length less than SNP length

107       total length vastly exceeds SNP length

108       truncated datagram with unadjusted checksum

109       truncated datagram with adjusted checksum

110       total length less than minimum IP header length

111       perfectly legal datagram

------------------------------

Test 100 version number too small

-Can the IUT detect a bad version number?

-Action:  Send a datagram from the IP Tester to the IP

Implementation Under Test whose sole defect is a version

number that is illegally small. This error should be caught

by the IUT. The IUT may send an ICMP Parameter Problem

message to   the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display should contain the string "ICMP

PARAMETER PROBLEM ID=00100". The string "ident = 100" should

not appear in the display of the response datagram because

this would indicate that the datagram was passed by the IUT

to its Remote Driver.

Success: An ICMP Parameter Problem message displayed by the

IP Tester. The total lack of an IP Tester response datagram

display is also an acceptable response.

Failure: A response datagram whose IP Tester display

contains the string "ident = 100".

Test 101 version number too large

-Can the IUT detect a bad version number?

-Action:  Send a datagram from the IP Tester to the IP

Implementation Under Test whose sole defect is a version

number that is illegally large. This error should be caught

by the IUT. The IUT may send an ICMP Parameter Problem

message to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display should contain the string "ICMP

PARAMETER PROBLEM ID=00101". The string "ident = 101" should

not appear in the display of the response datagram because

this would indicate that the datagram was passed by the IUT

to its Remote Driver.

Success: An ICMP Parameter Problem message displayed by the

IP Tester. The total lack of an IP Tester response datagram

display is also an acceptable response.

Failure: A response datagram whose IP Tester display

contains the string "ident = 101".

Test 102 internet header length too large

-Can the IUT detect a bad header length?

-Action:  Send a datagram from the IP Tester to the IP        -

Implementation Under Test whose sole defect is an internet

header length that is illegally large because it indicates

that the IP header exceeds the size of the IP datagram. This

error should be caught by the IUT. The IUT may send an ICMP

Parameter Problem message to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display should contain the string "ICMP

PARAMETER PROBLEM ID=00102". The string "ident = 102" should

not appear in the display of the response-datagram because

this would indicate that the datagram was passed by the IUT

to its Remote Driver.

Success: An ICMP Parameter Problem message displayed by the

IP Tester. The total lack of an IP Tester response datagram

display is also an acceptable response.

Failure: A response datagram whose IP Tester display

contains the string "ident = 102".

Test 103 internet header length too small

-Can the IUT detect a bad header length?

-Action:  Send a datagram from the IP Tester to the IP

Implementation Under Test whose sole defect is an internet

header length that is illegally small because it is less

than 5. This error should be caught by the IUT. The IUT may

send an ICMP Parameter Problem message to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display should contain the string "ICMP

PARAMETER PROBLEM ID=00103". The string "ident = 103" should

not appear in the display of the response datagram

because this would indicate that the datagram was passed by

the IUT to its Remote Driver.

Success: An ICMP Parameter Problem message displayed by the

IP Tester. The total lack of an IP Tester response datagram

display is also an acceptable response.

Failure: A response datagram whose IP Tester display

contains the string "ident = 103".

Test 104 internet header length equal to zero

-Can the IUT detect a bad header length?

-Action:  Send a datagram from the IP Tester to the IP

Implementation Under Test whose sole defect is an illegally

small internet header length of zero. This error should be

caught by the IUT. The IUT may send an ICMP Parameter

Problem message to the IP Tester.

This test differs from Test 103 because an internet header

length of zero is used in an attempt to induce an infinite

loop in the IUT when forming an ICMP Parameter Problem

message.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display should contain the string "ICMP

PARAMETER PROBLEM ID=00104". The string "ident = 104" should

not appear in the display of the response datagram because

this would indicate that the datagram was passed by the IUT

to its Remote Driver.

Infinite loop avoidance is indicated by a successful

response to Test 111, or Test 576 in Whole576Test if that

test is run last. In fact, any test that requires the IUT to

send a missive to the IP Tester may be used to indicate that

the IUT didn't get hung up in an infinite loop or crash.

Success: An ICMP Parameter Problem message displayed by the

IP Tester. The total lack of an IP Tester response datagram

display is also an acceptable response.

Failure: A response datagram whose IP Tester display

contains the string "ident = 104".

Test 105 total length exceeds actual length of IP datagram

-Can the IUT detect a bad total length?

-Action:  Send a datagram from the IP Tester to the IP

implementation Under Test whose sole defect is a total

length that indicates that the IP datagram is a byte longer

than it actually is. Unless informed otherwise, the lab

should consider this a section 9.4.6.2.7 error condition.

The IUT may send an ICMP Parameter Problem message to the IP

Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display should contain the string "ICMP

PARAMETER PROBLEM ID=00105". The string "ident = 105" should

not appear in the di;play of the response datagram because

this would indicate that the datagram was passed by the IUT

to its Remote Driver.

Success: An ICMP Parameter Problem message displayed by the

IP Tester. The total lack of an IP Tester response datagram

display is also an acceptable response.

Failure: A response datagram whose IP Tester display

contains the string "ident = 105".

Test 106 total length less than actual length of IP datagram

-Can the IUT detect a bad total length?

-Action:  Send a datagram from the IP Tester to the IP

Implementation Under Test whose sole defect is a total

length that indicates that the IP datagram is a byte shorter

than it actually is. Unless informed otherwise, the lab

should consider this a section 9.4.6.2.7 error condition.

The IUT may send an ICMP Parameter Problem message to the IP

Tester.

-Verification: If there is a response datagram displayed by

the IP Tester, its display should contain the string "ICMP

PARAMETER PROBLEM ID=OOl06". The string "ident = 106" should

not appear in the display of the response datagram because

this would indicate that the datagram was passed by the IUT

to its Remote Driver.

Success: An ICMP Parameter Problem message displayed by the

IP Tester. The total lack of an IP Tester response datagram

display is also an acceptable response.

WARNING: The IMPs on the DDN may append an extra data byte

in order to align the size of the message to a connected

computer's preferred word size. Passage of this test may

result in the inability to receive certain datagrams sent

over the DDN. The IMPs should be configured to deliver data

verbatim as implied by section 8.2.2.1.

Failure: A response datagram whose IP Tester display

contains the string "ident = 106".

Test 107 total length exceeds actual length of IP datagram

-Can the IUT detect a bad total length?

-Action: Send a datagram from the IP Tester to the IP

Implementation Under Test whose sole defect is a total

length that indicates that the IP datagram is vastly longer

than it actually is in an attempt to trigger a memory

management error or corrupt an implementation's data space.

Unless informed otherwise, the lab should consider this a

section 9.4.6.2.7 error condition. The IUT may send an ICMP

Parameter Problem message to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display should contain the string "ICMP

PARAMETER PROBLEM ID=00107". The string "ident = 107" should

not appear in the display of the response datagram because

this would indicate that the datagram was passed by the IUT

to its Remote Driver.

Success: An ICMP Parameter Problem message displayed by the

IP Tester. The total lack of an IP Tester response datagram

display is also an acceptable response.

Failure: A response datagram whose IP Tester display

contains the string "ident = 107".

Test 108 datagram length less than minimum and not multiple of 4

-Can the IUT handle a truncated datagram?.

-Action:  Send a datagram from the IP Tester to the IP

Implementation Under Test that is less than the minimum

legal length in an attempt to trigger a memory management

error or corrupt an implementation's data space. This is a

section 9.4.6.2.7 error condition. Because the checksum has

not been adjusted to be correct on the TRUNCATED header, the

checksum is incorrect. The IUT must discard this datagram

without notice.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 109 runt datagram with correct checksum

-Can the IUT handle a runt datagram?

-Action:  Send a datagram from the IP Tester to the IP

Implementation Under Test that is less than the minimum

legal length in an attempt to trigger a memory management

error or corrupt an implementation's data space. This is a

section 9.4.6.2.7 error condition. Because the checksum has

been adjusted to be correct on the TRUNCATED header, the IUT

may send an ICMP Parameter Problem message to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display should contain the string "ICMP

PARAMETER PROBLEM ID=00109".

Success: An ICMP Parameter Problem message displayed by the

IP Tester. The total lack of an IP Tester response datagram

display is also an acceptable response.

Failure: A failure to pass Test 111.

Test 110 datagram length less than minimum and multiple of 4

-Can the IUT handle a truncated datagram?

-Action:  Send a datagram from the IP Tester to the IP

Implementation Under Test that is less than the minimum

legal length in an attempt to trigger a memory management

error or corrupt an implementation's data space. This is a

section 9.4.6.2.7 error condition. Because the checksum has

not been adjusted on the TRUNCATED header, the checksum is

incorrect. The IUT must discard this datagram without

notice.

-Verification: The IP Tester should not display a response

datagram.

Don't take the response to the next test as the response to

this test. The tests follow back-to-back in rapid succession

to give an indication that the IUT is actually being tested.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 111 legal datagram

-Can the IUT handle a legal datagram?

-Action:  Send a datagram from the IP Tester to the IP

Implementation Under Test that is perfectly legal. This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should parse the Remote Driver

commands in the data field and request the IUT to send the

reply datagram required by the Remote Driver Specification

to the IP Tester.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 111 data" in its 16-byte data

field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

NOTICE: The failure of this test usually implies that one of

the previous tests has caused the IUT to enter an infinite

loop or crash.

Scenario File HdrChksumTest:  "IP Header Checksum Test" datagrams

800       header checksum one short

801       header checksum correct

802       header checksum are too large

803       header checksum one short with option

804       header checksum correct with option

805       header checksum one too large with option

-----------------------------------

Test 800 header checksum one less than it should be

-Can the IUT detect a bad checksum?

-Action:  Send a datagram from the IP Tester to the IP

Implementation Under Test whose sole defect is a header

checksum that is one less than it should be. The IUT must

discard this datagram without notice.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 801 datagram with correct checksum

-Can the IUT handle a datagram with good checksum?

-Action:  Send a datagram from the IP Tester to the IP

Implementation Under Test that is perfectly legal. This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should parse the Remote Driver

command in the data field and request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 801" in its 12-byte data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 802 header checksum one more than it should be

-Can the IUT detect a bad checksum?

-Action: Send a datagram from the IP Tester to the IP

Implementation Under Test whose sole defect is a header

checksum that is one more than it should be. The IUT must

discard this datagram without notice.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 803 header checksum one less than it should be plus option

-Can the IUT detect a bad checksum?

-Action:  Send a datagram containing an option from the IP

Tester to the IP Implementation Under Test whose sole defect

is a header checksum that is one less than it should be. The

IUT must discard this datagram without notice.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 804 datagram with correct checksum plus option

-Can the IUT handle a datagram with good checksum?

-Action:  Send a datagram containing an option from the IP

Tester to the IP Implementation Under Test that is perfectly

legal. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should parse the

Remote Driver command in the data field and request the IUT

to send the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 804" in its 12-byte data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 805 header checksum one more than it should be plus option

-Can the IUT detect a bad checksum?

-Action:  Send a datagram containing an option from the IP

Tester to the IP Implementation Under Test whose sole defect

is a header checksum that is one more than it should be. The

IUT must discard this datagram without notice.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Scenario File ICMPReasTest:  "ICMP Reassembly Tests"

2970      ICMP Echo Request message in fragments

2971      ICMP Echo Request message fragmented in reverse order

2972      fragmented ICMP Echo Request message and bogus

fragments

2973      fragmented ICMP Echo Request message with bad fragment

2974      normal datagram

-----------------------------------

Test 2970 ICMP Echo Request message in fragments

-Can the IUT reassemble an ICMP message?

-Action:  Send a fragmented ICMP Echo Request message to the

IP Implementation Under Test. The IUT should reassemble the

fragments and send an ICMP Echo Reply message to the IP

Tester.

-Verification: The IP Tester response datagram display

should include the string "ICMP ECHO REPLY id=00003". It

should also contain the string "TL00061" to indicate that

its total length is 61 bytes.

Success: An IP Tester ICMP Echo Reply datagram display.

Failure: Any response other than the display of the

requested ICMP Echo Reply message.

Test 2971 ICMP Echo Request message fragmented in reverse order

-Can the IUT reassemble an ICMP message fragmented in

reverse order?

-Action:  Send an ICMP Echo Request message fragmented in

reverse order to the IP Implementation Under Test. The IUT

should reassemble the fragments and send an ICMP Echo Reply

message to the IP Tester.

-Verification: The IP Tester response datagram display

should include the string "ICMP ECHO REPLY id=00004". It

should also contain the string "TLOO06l" to indicate that

its total length is 61 bytes.

Success: An IP Tester ICMP Echo Reply datagram display.

Failure: Any response other than the display of the

requested ICMP Echo Reply message.

Test 2972 fragmented ICMP Echo Request message and bogus

fragments

-Can the IUT reject bogus fragments?

-Action:  Send a fragmented ICMP Echo Request message and

bogus fragments to the IP Implementation Under Test. The

first bogus fragment has an incorrect version number which

should be discarded by the IUT. The second bogus fragment

has a time to live of one so it should be discarded by the

IUT. The third bogus fragment has an incorrect checksum and

should be discarded by the IUT. Each bogus fragment should

be discarded without notice because each is an ICMP

fragment. The IUT should reassemble the good fragments and

send an ICMP Echo Reply message to the IP Tester.

-Verification: The IP Tester response datagram display

should include the string "ICMP ECHO REPLY id=00005". It

should also contain the string "TLOO06l" to indicate that

its total length is 61 bytes.

Success: An IP Tester ICMP Echo Reply datagram display.

Failure: Any response other than the display of the

requested ICMP Echo Reply message.

Test 2973 fragmented ICMP Echo Request message with bad fragment

-Can the IUT discard ICMP fragment with bad time to live?

-Action: Send a fragmented ICMP Echo Request message to the

IP Implementation Under Test where one of the fragments has

a time to live of one. The bad fragment should be discarded

without notice and the other fragments should be discarded

without notice when the reassembly times out.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2974 normal datagram

-Can the IUT still handle a normal datagram?

-Action:  Send a datagram from the IP Tester to the IP

Implementation Under Test that is perfectly legal. This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should parse the Remote Driver

commands in the data field and request the IUT to send the

reply datagram required by the Remote Driver Specification

to the IP Tester.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2974" in its 13-byte data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

NOTICE: The failure of this test usually implies that one of

the previous tests has caused the IUT to enter an infinite

loop or crash.

Scenario File ICMPRedrctTest: "ICMP Redirection Tests"

2950      host redirection primer datagram

2951      ICMP host redirect to IP Tester

2952      positive test of ICMP host redirect

2953      ICMP host and type of service redirect to dead gateway

2954      negative test of ICMP host and type of service redirect

2955      host + host and type of service redirect independence

2956      ICMP host redirect to dead gateway

2957      ICMP host and type of service redirect to IP Tester

2958      negative test of ICMP host redirect

2959      positive test of ICMP host and type of service redirect

2960      network redirection primer datagram

2961      ICMP network redirect to IP Tester

2962      positive test of ICMP network redirect

2963      network and type of service redirect to dead gateway

2964      negative test of network and type of service redirect

2965      net + net and type of service redirect independence

2966      ICMP network redirect to dead gateway

2967      ICMP network and type of service redirect to IP Tester

2968      negative test of ICMP network redirect

2969      positive test of network and type of service redirect

-----------------------------------

Test 2950 ICMP host redirection primer datagram

-Datagram to prepare an implementation for host redirection.

RFC792 pages 12-13 state that an ICMP redirect message is

sent in response to a datagram. The implementation is asked

to send a datagram to provide cause.

-Action:  Send an ordinary datagram from the IP Tester to

the IP Implementation Under Test that is perfectly legal.

This datagram should be passed by the IUT to its Remote

Driver. The IUT's Remote Driver should parse the Remote

Driver command in the data field and request the IUT to send

the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2950" in its 13-byte data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 2951 ICMP host redirection

-Can the IUT redirect a datagram for a host?

-Action:  An ICMP Host Redirect message is sent to the IUT

by the IP Tester. The message is based on the datagram that

the IUT is required to send in Test 2950. The ICMP Host

Redirect message instructs the IUT to update its routing

table(s) to send datagrams to intended for "host X" to the

IP Tester. The IUT is ready to be tested to see if it

handles the message.

-Verification: The IP Tester should not display a response

datagram. Actual verification of host redirection occurs in

Tests 2952 and 2955.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2952 ICMP host redirection verification

-Can the IUT redirect a datagram for a host?

-Action:  Send a datagram to the IP Implementation Under

Test. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the Remote Driver

Specification to the IP Tester since the IUT was told to

redirect datagrams for "host X" in Test 2951.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2952 dest = 1711341568" in its

31-byte data field.

Success: Display of reply datagram.

Failure: Any response other than the display of the

requested reply datagram.

Test 2953 ICMP host and type of service redirection

-Can the IUT redirect a datagram for a host based on its

type of service?

-Action:  An ICMP Host and Type of Service Redirect message

is sent to the IUT by the IP Tester. The message is based on

the datagram that the IUT is required to send in Test 2950.

The message contains a precedence of 1 in the "copy" of the

"original" datagram's header field. The ICMP Host and Type

of Service Redirect message instructs the IUT to update its

routing table(s) to send datagrams with precedence 1

intended for "host X" to the "dead gateway," an unused port

on one of the EDN's IMPs. The IUT is ready to be tested to

see if it handles the message.

-Verification: The IP Tester should not display a response

datagram. Actual verification of host and type of service

redirection occurs in Test 2954.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2954 ICMP host and type of service redirection verification

-Can the IUT redirect a datagram for a host based on type of

service?

-Action:  Send a datagram to the IP Implementation Under

Test. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the Remote Driver

Specification to the IUT because the IUT was told to

redirect datagrams with precedence of 1 for "host X" in Test

2953. The IUT should send the reply datagram to the "dead

gateway." The IMP should discard it without notice on the IP

level.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2955 ICMP host redirection additional verification

-Can the IUT remember host redirection after host and type

of service redirection?

-Action:  Send a datagram to the IP Implementation Under

Test. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver shouId request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester because the IUT was

told to redirect datagrams for "host X" in Test 2951.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2955 dest = 1711341568" in its

31-byte data field.

Success: Display of reply datagram.

Failure: Any response other than the display of the

requested reply datagram.

Test 2956 ICMP host redirection

-Can the IUT redirect a datagram for a host?

-Action:  An ICMP Host Redirect message is sent to the IUT

by the IP Tester. The message is based on the datagram that

the IUT is required to send in Test 2950. The ICMP Host

Redirect message instructs the IUT to update its routing

table(s) to send datagrams to intended for "host X" to the

"dead gateway."

-Verification: The IP Tester should not display a response

datagram. Actual verification of host redirection occurs in

Test 2958.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2957 ICMP host and type of service redirection

-Can the IUT redirect a datagram for a host based on its

type of service?

-Action:  An ICMP Host and Type of Service Redirect message

is sent to the IUT by the IP Tester. The message is based on

the datagram that the IUT is required to send in Test 2950.

The message contains a precedence of 1 in the "copy" of the

"original" datagram's header field. The ICMP Host and Type

of Service Redirect message instructs the IUT to update its

routing table(s) to send datagrams with precedence 1

intended for "host X" to the IP Tester.

-Verification: The IP Tester should not display a response

datagram. Actual verification of host and type of service

redirection occurs in Test 2959.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2958 ICMP host redirection verification

-Can the IUT redirect a datagram for a host?

-Action:  Send a datagram to the IP Implementation Under

Test. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the Remote Driver

Specification to the IUT because the IUT was told to

redirect datagrams for "host X" in Test 2956. The IUT should

send the reply datagram to the "dead gateway." The IMP

should discard it without notice on the IP level.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2959 ICMP host and type of service redirection verification

-Can the IUT redirect a datagram for a host based on type of

service?

-Action:  Send a datagram to the IP Implementation Under

Test. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the Remote Driver

Specification to the IUT because the IUT was told to

redirect datagrams with precedence of 1 for "host X" in Test

2957. The IUT should send the reply datagram to the IP

Tester.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2959 dest = 1711341568" in its

31-byte data field.

Success: Display of reply datagram.

Failure: Any response other than the display of the

requested reply datagram.

Test 2960 ICMP network redirection primer datagram

-Datagram to prepare an implementation for network

redirection.

RFC792 pages 12-13 state that an ICMP redirect message is

sent in response to a datagram. The implementation is asked

to send a datagram to provide cause.

-Action:  Send an ordinary datagram from the IP Tester to

the IP Implementation Under Test that is perfectly legal.

This datagram should be passed by the IUT to its Remote

Driver. The IUT's Remote Driver should parse the Remote

Driver command in the data field and request the IUT to send

the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2960" in its 13-byte data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 2961 ICMP network redirection

-Can the IUT redirect a datagram for a network?

-Action: An ICMP Network Redirect message is sent to the IUT

by the IP Tester. The message is based on the datagram that

the IUT is required to send in Test 2960. The ICMP Network

Redirect message instructs the IUT to update its routing

table(s) to send datagrams to intended for "network Y" to

the IP Tester. The IUT is ready to be tested to see if it

handles the message.

-Verification: The IP Tester should not display a response

datagram. Actual verification of network redirection occurs

in Tests 2962 and 2965.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2962 ICMP network redirection verification

-Can the IUT redirect a datagram for a network?

-Action:  Send a datagram to the IP Implementation Under

Test. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the Remote Driver

Specification to the IP Tester because the IUT was told to

redirect datagrams for "network Y" in Test 2961.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2962 dest = 1728118784" in its

31-byte data field.

Success: Display of reply datagram.

Failure: Any response other than the display of the

requested reply datagram.

Test 2963 ICMP network and type of service redirection

-Can the IUT redirect a datagram for a network based on its

type of service?

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2965 dest = 1728118784" in its

31-byte data field.

Success: Display of reply datagram.

Failure: Any response other than the display of the

requested reply datagram.

Test 2966 ICMP network redirection

-Can the IUT redirect a datagram for a network?

-Action:  An ICMP Network Redirect message is sent to the

IUT by the IP Tester. The message is based on the datagram

that the IUT is required to send in Test 2960. The ICMP

Network Redirect message instructs the IUT to update its

routing table(s) to send datagrams to intended for "network

Y" to the"dead gateway."

-Verification: The IP Tester should not display a response

datagram. Actual verification of network redirection occurs

in Test 2968.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.-

Test 2967 ICMP network and type of service redirection

-Can the IUT redirect a datagram for a network based on its

type of service?

-Action:  An ICMP Network and Type of Service Redirect

message is sent to the IUT by the IP Tester. The message is

based on the datagram that the IUT is required to send in

Test 2960. The message contains a precedence of 1 in the

"copy" of the "original" datagram's header field. The ICMP

Network and Type of Service Redirect message instructs the

IUT to update its routing table(s) to send datagrams with

precedence 1 intended for "network Y" to the IP Tester.

-Verification: The IP Tester should not display a response

datagram. Actual verification of network and type of service

redirection occurs in Test 2969.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2968 ICMP network redirection verification

-Can the IUT redirect a datagram for a network?

-Action:  Send a datagram to the IP Implementation Under

Test. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the Remote Driver

Specification to the IUT because the IUT was told to

redirect datagrams for "network Y" in Test 2966. The IUT

should send the reply datagram to the "dead gateway." The

IMP should discard it without notice on the IP level.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2969 network and type of service redirection verification

-Can the IUT redirect a datagram for a network based on type

of service?

-Action:  Send a datagram to the IP Implementation Under

Test. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the Remote Driver

Specification to the IUT because the IUT was told to

redirect datagrams with precedence of 1 for "network Y" in

Test 2967. The IUT should send the reply datagram to the IP

Tester.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2969 dest = 1728118784" in its

31-byte data field.

Success: Display of reply datagram.

Failure: Any response other than the display of the

requested reply datagram.

Scenario File ICMPSrcQueTest: "ICMP Source Quench Test" 2930-2934

ICMP Source Quench

Test 2930 ICMP Source Quench primer datagram

-Datagram to prepare IUT to be source quenched.

Section 9.4.6.3.1 states that an ICMP Source Quench message

indicates that a datagram has been discarded. This implies

that a datagram was sent by the implementation. Protocol

implementations for a military environment may not allow a

random message to impede their operation for even a short

time without cause. The implementation is asked send a

datagram to provide cause.

It is unlikely that an implementation will check unless

required to by contract.

-Action:  Send an ordinary datagram from the IP Tester to

the IP Implementation Under Test that is perfectly legal.

This datagram should be passed by the IUT to its Remote

Driver. The IUT's Remote Driver should parse the Remote

Driver command in the data field and request the IUT to send

the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2930 " in its 13-byte data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 2931 first ICMP Source Quench

-Can the IUT shut up or slow down when source quenched?

-Action:  An ICMP Source Quench message is sent to the IUT

by the IP Tester. The message is based on the datagram that

the IUT is required to send in Test 2930. The ICMP Source

Quench message instructs the IUT or its upper level protocol

(its Remote Driver) to reduce the rate at which it is

sending datagrams. Because the IP Remote Driver is not

required by the IP Remote Driver Specification to process an

ICMP Source Quench, the IUT is ready to be tested to see if

it handles the message.

Eight datagrams are sent to the IUT by the IP Tester. These

should by accepted by the IUT and passed to its Remote

Driver. The Remote Driver should request the IUT to send a

reply datagram for each one. The IUT should eventually send

the eight reply datagrams to the IP Tester.

-Verification: The IP Tester will display the eight reply

datagrams. Each must contain "ident = 780 " in its 12-byte

data field. Because the script asynchronously instructs the

IP Test Controller to write "tick" to the log file every

second, the displays may be a bit messy. This is normal and

desirable because it provides the means to determine how

much of a rate reduction the IUT is providing.

There is no official performance criterion.

Success: Display of each of the eight reply datagrams

requested to be sent.

Failure: Any response other than the display of the

requested reply datagrams.

WARNING: The IUT may forget that it sent the datagram in

Test 2930, causing the Source Quench to be ignored. This can

be counteracted by adjusting the wait time (wait 10) for

Test 2930 to a smaller value. This problem is unlikely.

Test 2932 second ICMP Source Quench

-Can the IUT further reduce its rate of transmission?

-Action:  An ICMP Source Quench message is sent to the IUT

by the IP Tester. The message is based on the datagram that

the IUT is required to send in Test 2930.

Eight datagrams are sent to the IUT by the IP Tester. These

should by accepted by the IUT and passed to its Remote

Driver. The Remote Driver should request the IUT to send a

reply datagram for each one. The IUT should eventually send

the eight reply datagrams to the IP Tester.

-Verification: The IP Tester will display the eight reply

datagrams. Each must contain "ident = 780 " in its 12-byte

data field.

There is no official performance criterio~. However, the

Internet Control Message Protocol Protocol Specification

TM-WD-7411/300/01 states "the source host should cut back

the rate at which it is sending traffic to the specified

destination." This can be confirmed by fewer characters (on

average) between "ticks." MIL-STD-1777 only requires that

"the ULP should be informed so traffic can be reduced."

Therefore, the lack of FURTHER reduction isn't non-

conformant. However, all known ICMP standards require

reduction.

Success: Display of each of the eight reply datagrams

requested to be sent.

Failure: Any response other than the display of the

requested reply datagrams.

Test 2933 third ICMP Source Quench

-Can the IUT further reduce its rate of transmission?

-Action:  An ICMP Source Quench message is sent to the IUT

by the IP Tester. The message is based on the datagram that

the IUT is required to send in Test 2930.

Eight datagrams are sent to the IUT by the IP Tester. These

should by accepted by the IUT and passed to its Remote

Driver. The Remote Driver should request the IUT to send a

reply datagram for each one. The IUT should eventually send

the eight reply datagrams to the IP Tester.

-Verification: The IP Tester will display the eight reply

datagrams. Each must contain "ident = 780 " in its 12-byte

data field.

There is no official performance criterion. However, the

Internet Control Message Protocol Protocol Specification

TM-WD-7411/300/01 states "the source host should cut back

the rate at which it is sending traffic to the specified

destination." This can be confirmed by fewer characters (on

average) between "ticks." MIL-STD-1777 only requires that

"the ULP should be informed so traffic can be reduced."

Therefore, the lack of FURTHER reduction isn't non-

conformant. However, all known ICMP standards require

reduction.

Success: Display of each of the eight reply datagrams

requested to be sent.

Failure: Any response other than the display of the

requested reply datagrams.

Test 2934 fourth ICMP Source Quench

-Can the IUT further reduce its rate of transmission?

-Action:  An ICMP Source Quench message is sent to the IUT

by the IP Tester. The message is based on the datagram that

the IUT is required to send in Test 2930.

Eight datagrams are sent to the IUT by the IP Tester. These

should by accepted by the IUT and passed to its Remote

Driver. The Remote Driver should request the IUT to send a

reply datagram for each one. The IUT should eventually send

the eight reply datagrams to the IP Tester.

-Verification: The IP Tester will display the eight reply

datagrams. Each must contain "ident = 780 " in its 12-byte

data field.

There is no official performance criterion. However, the

Internet Control Message Protocol Protocol Specification

TM-WD-7411/300/01 states "the source host should cut back

the rate at which it is sending traffic to the specified

destination.' This can be confirmed by fewer characters (on

average) between "ticks." MIL-STD-1777 only requires that

"the ULP should be informed so traffic can be reduced.'

Therefore, the lack of FURTHER reduction isn't non-

conformant.   However, all known ICMP standards require

reduction.

Success: Display of each of the eight reply datagrams

requested to be sent.

Failure: Any response other than the display of the

requested reply datagrams.

NOTICE: The IUT could conceivably send an ICMP Source Quench

message, whose display will contain the string "unexpected

ICMP error type=4". MIL-STD-1777 does not indicate how this

is to be interpreted. The laboratory will not issue a report

on the IUT until the specification is amended. IUTs having

difficulties will have to wait (2+ years ?) for

certification.

Scenario File ICMPartlTest:  "Miscellaneous ICMP Tests"

2900      normal datagram

2901      ICMP echo request with bad IP header checksum

2902      ICMP echo request with bad ICMP header checksum

2903      valid ICMP echo request

2904      ICMP Parameter Problem

2905      ICMP time to live exceeded

2906      ICMP reassembly timeout

2907      ICMP unreachable network

2908      ICMP unreachable host

2909      ICMP unreachable protocol

2910      ICMP unreachable port

2911      ICMP fragmentation needed and DF set

2912      ICMP source route failed

2913      normal datagram

-----------------------------------

Test 2900 ICMP message primer datagram

-Datagram to prepare an implementation for ICMP messages.

-Action:  Send an ordinary datagram from the IP Tester to

the IP Implementation Under Test that is perfectly legal.

This datagram should be passed by the IUT to its Remote

Driver. The IUT's Remote Driver should parse the Remote

Driver command in the data field and request the IUT to send

the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2900 " in its 13-byte data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 2901 ICMP Echo Request message with bad IP header chec

-Can the IUT detect a bad IP header checksum in an ICMP

message?

-Action:  Send an ICMP Echo Request message to the IP

Implementation Under Test whose sole defect is a bad IP

header checksum. The IUT must discard this datagram without

notice.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2902 ICMP Echo Request message with bad ICMP checksum

-Can the IUT detect a bad ICMP checksum?

-Action:  Send an ICMP Echo Request message to the IP

Implementation Under Test whose sole defect is a bad ICMP

checksum. The IUT should discard this datagram without

notice.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2903 ICMP Echo Request message with valid checksums

-Can the IUT reply to an ICMP Echo Request message?

-Action:  Send a legal ICMP Echo Request message to the IP

Implementation Under Test. The IUT should send an ICMP Echo

Reply message to the IP Tester.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2907 ICMP Unreachable Network message

-Can the IUT handle an ICMP Unreachable Network message?

-Action:  Send an ICMP Unreachable Network message to the IP

-Implementation Under Test. All further action is

implementation dependent according to section 9.4.6.3.1. The

most likely action is to log the message.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2908 ICMP Unreachable Host message

-Can the IUT deal with an ICMP Unreachable Host message?

-Action:  Send an ICMP Unreachable Host message to the IP

Implementation Under Test. All further action is

implementation dependent according to section 9.4.6.3.1. The

most likely action is to log the message.

-Verification: The IP-Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2909 ICMP Unreachable Protocol message

-Can the IUT handle an ICMP Unreachable Protocol message?

-Action:  Send an ICMP Unreachable Protocol message to the

IP Implementation Under Test. All further action is

implementation dependent according to section 9.4.6.3.1. The

most likely action is to log the message.

-Verification: The IP Tester should not display a response

datagram.

-Verification: The IP Tester response datagram display

should include the string "ICMP ECHO REPLY id=00002". It

should also contain the string "TL00046"' to indicate that

its total length is 46 bytes.

Success: An IP Tester ICMP Echo Reply datagram display.

Failure: Any response other than the display of the

requested ICMP Echo Reply message.

Test 2904 ICMP Parameter Problem message

-Can the IUT deal with an ICMP Parameter Problem message?

-Action:  Send an ICMP Parameter Problem message to the IP

Implementation Under Test. All further action is

implementation dependent according to section 9.4.6.3.1. The

most likely action is to log the message.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2905 ICMP Time to Live Exceeded In Transit message

-Can the IUT deal with an ICMP Time Exceeded message?

-Action:  Send an ICMP Time Exceeded in Transit message to

the IP Implementation Under Test. All further action is

implementation dependent according to section 9.4.6.3.1. The

most likely action is to log the message.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2906 ICMP Time to Live Exceeded During Reassembly message

-Can the IUT deal with an ICMP Time Exceeded message?

-Action:  Send an ICMP Time Exceeded During Reassembly

message to the IP Implementation Under Test. All further

action is implementation dependent according to section

9.4.6.3.1. The most likely action is to log the message.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2910 ICMP Unreachable Port message

-Can the IUT deal with an ICMP Unreachable Port message?

-Action:  Send an ICMP Unreachable Port message to the IP

Implementation Under Test. All further action is

implementation dependent according to section 9.4.6.3.1. The

most likely action is to log the message.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2911 ICMP Fragmentation Needed and DF Set message

-Can the IUT deal with an ICMP Fragmentation Needed and DF

Set message?

-Action:  Send an ICMP Fragmentation Needed and DF Set-

message to the IP Implementation Under Test. All further

action is implementation dependent according to section

9.4.6.3.1. The most likely action is to log the message.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2912 ICMP Source Route Failed message

-Can the IUT deal with an ICMP Source Route Failed message?

-Action:  Send an ICMP Source Route Failed message to the IP

Implementation Under Test. All further action is

implementation dependent according to section 9.4.6.3.1. The

most likely action is to log the message.

-Verification: The IP Tester should not display a response

datagram.

Success: The total lack of an IP Tester response datagram

display is the only acceptable response.

Failure: An IP Tester response datagram display of any kind.

Test 2913 ICMP message robustness check

-Did the IUT handle bogus ICMP messages without crashing?

-Action:  Send a datagram to the IP Implementation Under

Test. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the Remote Driver

Specification to the IP Tester.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2913 " in its 13-byte data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram. The actual failure probably

occurred during one of the previous tests.

Scenario File IdentTest: "Identification Field Test' datagrams

0         send and receive fields 0

65535     send and receive fields 65535

3690      send and receive fields 43690

21845     send and receive fields 21845

65535     send field 65535 and receive field 0

0         send field 0 and receive field 65535

21845     send field 21845 and receive field 43690

43690     send field 43690 and receive field 21845

-----------------------------------

Test 0 send and receive identification fields of 0

-Can the IUT receive and send a datagram with an ID of 0?

-Action:  Send a datagram to the IP Implementation Under

Test with an identification field of 0. This datagram should

be passed by the IUT to its Remote Driver. The IUT's Remote

Driver should request the IUT to send the reply datagram

required by the IP Remote Driver Specification to the IP

Tester. The reply datagram ought to have an identification

field of 0.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 0 " in its 10-byte data field. In

addition, the display ought to contain the string "ID00000",

but any value may be used instead of "00000."

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 65535 send and receive identification fields of 65535

-Can the IUT receive and send a datagram with an ID of

65535?

-Action:  Send a datagram to the IP Implementation Under

Test with an identification field of 65535. This datagram

should be passed by the IUT to its Remote Driver. The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester. The reply datagram ought to have an

identification field of 65535.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 65535" in its 14-byte data field.

In addition, the display ought to contain the string

"ID65535", but any value may be used instead of "65535."

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 43690 send and receive identification fields of 43690

-Can the IUT receive and send a datagram with an ID of

43690?

-Action:  Send a datagram to the IP Implementation Under

Test with an identification field of 43690 (odd-numbered

bits set). This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester. The reply datagram

ought to have an identification field of 43690.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 43690' in its 14-byte data field.

In addition, the display ought to contain the string

"ID43690', but any value may be used instead of "43690.'

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 21845 send and receive indentification fields of 21845

-Can the IUT receive and send a datagram with an ID of

21845?

-Action:  Send a datagram to the IP Implementation Under

Test with an identification field of 21845 (even-numbered

bits set). This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester. The reply datagram

ought to have an identification field of 21845.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 21845 " in its 14-byte data

field. In addition, the display ought to contain the string

"ID21845', but any value may be used instead of "21845.'

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 65535 send field of 65535 and receive field of 0

-Can the IUT accept an identification field from its upper

level protocol?

This test prevents IUTs from recording the identification

field of the datagram passed to the Remote Driver and using

it in the reply datagram sent by the Remote Driver to gain a

better report.

-Action:  Send a datagram to the IP Implementation Under

Test with an identification field of 0. This datagram should

be passed by the IUT to its Remote Driver. The IUT's Remote

Driver should request the IUT to send the reply datagram

required by the IP Remote Driver Specification to the IP

Tester. The reply datagram ought to have an identification

field of 65535.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 65535 " in its 14-byte data

field. In addition, the display ought to contain the string

"ID65535", but any value may be used instead of "65535."

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 0 send field of 0 and receive field of 65535

-Can the IUT accept an identification field from its upper

level protocol

-Action:  Send a datagram to the IP Implementation Under

Test with an identification field of 65535. This datagram

should be passed by the IUT to its Remote Driver. The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester. The reply datagram ought to have an

identification field of 0.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 0 " in its 10-byte data field. In

addition, the display ought to contain the string "ID00000",

but any value may be used instead of "00000."

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 21845 send field of 21845 and receive field of 43690

-Can the IUT accept an identification field from its upper

level protocol

-Action:  Send a datagram to the IP Implementation Under

Test with an identification field of 43690. This datagram

should be passed by the IUT to its Remote Driver. The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester. The reply datagram ought have an

identification field of 21845.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 21845 " in its 14-byte data

field. In addition, the display ought to contain the string

"ID21845", but any value may be used instead of "21845."

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 43690 send field of 43690 and receive field of 21845

-Can the IUT accept an identification field from its upper

level protocol ?

-Action:  Send a datagram to the IP Implementation Under

Test with an identification field of 21845. This datagram

should be passed by the IUT to its Remote Driver. The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester. The reply datagram ought to have an

identification field of 43690.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 43690 " in its 14-byte data

field. In addition, the display ought to contain the string

"ID43690", but any value may be used instead of "43690."

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Scenario File NoOpLenBotTest: "Bottom-up Missing Option Length"

datagrams

2200      security

2201      stream identification

2202      record route

2203      timestamp

2204      loose source

2205      strict source

-----------------------------------

Test 2200 security option sans option length

-Can the IUT interpret an incomplete security option?

-Action:  Send a datagram to the IP Implementation Under

Test with a 4-byte option field. The first three bytes are

no operation options (to permit the truncation of the

following "option"). The last byte represents the option

type byte of a security option. Normally, the option type is

followed by the option length. The IUT is required to figure

out the proper action in accordance with section 9.1.

Because there is not very much to "interpret," the IUT

probably should send an ICMP Parameter Problem message to

the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02200". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2200 " in its 13-byte data field.

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2200", or nothing.

Failure: A failure to pass the next test performed after

this test that requires a positive response from the IUT

(Test 4200).

Test 2201 stream identification option sans option length

-Can the IUT interpret an incomplete stream ID option?

-Action:  Send a datagram to the IP Implementation Under

Test with a 4-byte option field. The first three bytes are

no operation options (to permit the truncation of the

following "option"). The last byte represents the option

type byte of a stream ID option. Normally, the option type

is followed by the option length. The IUT is required to

figure out the proper action in accordance with section 9.1.

Because there is not very much to "interpret," the IUT

probably should send an ICMP Parameter Problem message to

the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote-Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02201". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2201 " in its 13-byte data field.

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2201 ", or nothing.

Failure: A failure to pass the next test performed after

this test that requires a positive response from the IUT

(Test 4200).

Test 2202 record route option sans option length

-Can the IUT interpret an incomplete record route option?

-Action:  Send a datagram to the IP Implementation Under

Test with a 4-byte option field. The first three bytes are

no operation options (to permit the truncation of the

following "option"). The last byte represents the option

type byte of a record route option. Normally, the option

type is followed by the option length. The IUT is required

to figure out the proper action in accordance with section

9.1. Because there is not very much to "interpret," the IUT

probably should send an ICMP Parameter Problem message to

the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02202". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2202" in its 13-byte data field.

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2202 ", or nothing.

Failure: A failure to pass the next test performed after

this test that requires a positive response from the IUT

(Test 4200).

Test 2203 timestamp option sans option length

-Can the IUT interpret an incomplete timestamp option?

-Action:  Send a datagram to the IP Implementation Under

Test with a 4-byte option field. The first three bytes are

no operation options (to permit the truncation of the

following "option.") The last byte represents the option

type byte of a timestamp option. Normally, the option type

is followed by the option length. The IUT is required to

figure out the proper action in accordance with section 9.1.

Because there is not very much to "interpret," the IUT

probably should send an ICMP Parameter Problem message to

the IP Tester and discard the datagram.

It is possible that the IUT' will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification: If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02203". However, if the IUT chooses to

pass the datagram to its Remote Driver the reply datagram

must contain "ident = 2203 " in its 13-byte data field.

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2203 ", or nothing.

Failure: A failure to pass the next test performed after

this test that requires a positive response from the IUT

(Test 4200).

Test 2204 loose source and record route option sans option length

-Can the IUT interpret an incomplete loose source and record

route option?

-Action:  Send a datagram to the IP Implementation Under

Test with a 4-byte option field. The first three bytes are

no operation options (to permit the elimination of the

option length). The last byte represents the option type

byte of a loose source and record route option. Normally,

the option type is followed by the option length. The IUT is

required to figure out the proper action in accordance with

section 9.1. Because there is not very much to "interpret,"

the IUT probably should send an ICMP Parameter Problem

message to the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02204". However, if the IUT chooses to

pass the datagram to Its Remote Driver, the reply datagram

must contain "ident = 2204 " in its 13-byte data field.

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2204", or nothing.

Failure: A failure to pass the next test performed after

this test that requires a positive response from the IUT

(Test 4200).

Test 2205 strict source and record route option sans option

length

-Can the IUT interpret an incomplete strict source and

record route option?

-Action:  Send a datagram to the IP Implementation Under

Test with a 4-byte option field. The first three bytes are

no operation options (to permit the truncation of the

following "option"). The last byte represents the option

type byte of a strict source and record route option.

Normally, the option type is followed by the option length.

The IUT is required to figure out the proper action in

accordance with section 9.1.  Because there is not very much

to "interpret," the IUT probably should send an ICMP

Parameter Problem message to the IP Tester and discard the

datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02205". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2205 " in its 13-byte data field.

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2205 ", or nothing.

Failure: A failure to pass the next test performed after

this test that requires a positive response from the IUT

(Test 4200).

Scenario File NoOpLenTopTest: "Top-down Missing Option

Length" datagrams

4200      security

4201      stream identification

4202      record route

4203      timestamp

4204      loose source

4205      strict source

-----------------------------------

Test 4200 security option sans option length

-Can the IUT send an incomplete security option?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a 4-byte option field. This datagram

should be passed by the IUT to its Remote Driver. The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester. The first three bytes of the invalid option

field that the IUT is requested to send are no operation

options (to permit the truncation of the following

"option").   The last byte represents the option type byte of

a security option. The IUT is required to inform its Remote

Driver that the request is invalid. The Remote Driver should

then request the IUT to send an IP Remote Driver

Specification error message to the IP Tester.

-Verification: The IP Tester will display the error message

datagram that must contain "ident = 4200 error" in its 18-

byte data field. The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success: Display of error message datagram.

Failure: Any response other than the display of the error

message datagram.

Test 4201 stream ID option sans option length

-Can the IUT send an incomplete stream ID option?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a 4-byte option field. This datagram

should be passed by the IUT to its Remote Driver. The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver-Specification to

the IP Tester. The first three bytes of the invalid option

field that the IUT is requested to send are no operation

options (to permit the truncation of the following

"option"). The last byte represents the option type byte of

a stream identification option. The IUT is required to

inform its Remote Driver that the request is invalid. The

Remote Driver should then request the IUT to send an IP

Remote Driver Specification error message to the IP Tester.

-Verification: The IP Tester will display the error message

datagram that must contain "ident = 4201 error" in its 18-

byte data field. The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success: Display of error message datagram.

Failure: Any response other than the display of the error

message datagram.

Test 4202 record route option sans option length

-Can the IUT send an incomplete record route option?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a 4-byte option field. This datagram

should be passed by the IUT to its Remote Driver. The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester. The first three bytes of the invalid option

field that the IUT is requested to send are no operation

options (to permit the truncation of the following

"option").  The last byte represents the option type byte of

a record route option. The IUT is required to inform its

Remote Driver that the request is invalid. The Remote Driver

should then request the IUT to send an IP Remote Driver

Specification error message to the IP Tester.

-Verification: The IP Tester will display the error message

datagram that must contain "ident = 4202 error" in its 18-

byte data field. The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success: Display of error message datagram.

Failure: Any reaponse other than the display of the error

message datagram.

Test 4203 timestamp option sans option length

-Can the IUT send an incomplete record route option?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a 4-byte option fieId. This datagram

should be passed by the IUT to its Remote Driver. The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester. The first three bytes of the invalid option

field that the IUT is requested to send are no operation

options (to permit the truncation of the following

"option").  The last byte represents the option type byte of

a timestamp option. The IUT is required to inform its Remote

Driver that the request is invalid. The Remote Driver should

then request the IUT to send an IP Remote Driver

Specification error message to the IP Tester.

-Verification: The IP Tester will display the error message

datagram that must contain "ident = 4203 error" in its 18-

byte data field. The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success: Display of error message datagram.

Failure: Any response other than the display of the error

message datagram.

Test 4204 loose source and record route option sans option length

-Can the IUT send an incomplete loose source and record

route option?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a 4-byte option field. This datagram

should be passed by the IUT to its Remote Driver. The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester. The first three bytes of the invalid option

field that the IUT is requested to send are no operation

options (to permit the truncation of the following

"option"). The last byte represents the option type byte of

a loose source and record route option. The IUT is required

to inform its Remote Driver that the request is invalid. The

Remote Driver should then request the IUT to send an IP

Remote Driver Specification error message to the IP Tester.

-Verification: The IP Tester will display the error message

datagram that must contain "ident = 4204 error" in its 18-

byte data field. The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success: Display of error message datagram.

Failure: Any response other than the display of the error

message datagram.

Test 4205 strict source and record route option sans option

length

-Can the IUT send an incomplete strict source and record

route option?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a 4-byte option field. This datagram

should be passed by the IUT to its Remote Driver. The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester. The first three bytes of the invalid option

field that the IUT is requested to send are no operation

options (to permit the truncation of the following

"option"). The last byte represents the option type byte of

a strict source and record route option. The IUT is required

to inform its Remote Driver that the request is invalid. The

Remote Driver should then request the IUT to send an IP

Remote Driver Specification error message to the IP Tester.

-Verification: The IP Tester will display the error message

datagram that must contain "ident = 4205 error" in its 18-

byte data field. The IP Tester should also display "HL5"

(internet leader length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success: Display of error message datagram.

Failure: Any response other than the display of the error

message datagram.

Scenario File OddReasTest: "Oddball Reassembly Tests"

570       MF and DF both set

571       DF set and fragment offset not 0

572       enormous fragment offset

573       range of fragment offsets

574       precedence mismatch

575       security level mismatch with unclassified exception

576       security level mismatch

577       security level mismatch with classified exception

578       first compartment byte mismatch

579       second compartment byte mismatch

580       first handling byte mismatch

581       second handling byte mismatch

582       first transmission control code byte mismatch

583       second transmission control code byte mismatch

584       third transmission control code byte mismatch

585       security option present exception

-----------------------------------

Test 570  more and don't fragment flags both set

-Can the IUT deal with contradictory framentation flags?

-Action:  Send a datagram to the IP Implementation Under

Test.with both the more fragment(s) and don't fragment flags

set. There are two ways the IUT could choose to handle the

contradictory flags.

One method is to create a state vector and record the

"fragment." When the state vector's timer expires, the state

vector and "fragment" will be discarded and an ICMP

reassembly timeout message may be sent by the IUT to the IP

Tester. Another method envisioned is that the IUT will

detect that both of the fragmentation flags are set. The IUT

may then send an ICMP Parameter Problem message to the IP

Tester. This method should be considered legal by the lab

under section 9.4.6.2.7, unless otherwise instructed.

-Verification:  If there is an IP Tester response datagram

display, it should contain either "ICMP TIME EXCEEDED

id=00570 REAS" or "ICMP PARAMETER PROBLEM id=00570". The

response datagram display should not contain the string

"ident" because that would indicate that the "fragment" was

passed to the IUT's Remote Driver. The total lack of a

response datagram is also a correct response.

Success: An ICMP reassembly timeout message, ICMP Parameter

Problem message, or nothing.

Failure: A reply datagram containing "ident".

Test 571 missive with non-zero offset with don't fragment flag

set

-Can the IUT deal with a missive with contradictory fragment

offset and don't fragment flag fields?

-Action:  Send a datagram to the IP Implementation Under

Test with a non-zero fragment offset and the don't fragment

flag set. There are two ways the IUT could choose to handle

the contradictory fields.

One method is to create a state vector and record the

"fragment." When the state vector's timer expires, the state

vector and "fragment" will be discarded and an ICMP

reassembly timeout message may be sent by the IUT to the IP

Tester.

Another method envisioned is that the IUT will detect that

fields are contradictory. The IUT may then send an ICMP

Parameter Problem to the IP Tester. Unless instructed

otherwise, this method should be considered legal by the lab

under section 9.4.6.2.7.

-Verification:  If there is an IP Tester response datagram

display, it should contain either "ICMP TIME EXCEEDED

id=00571 REAS" or "ICMP PARAMETER PROBLEM id=00571". The

response datagram display should not contain the string

"ident" because that would indicate that the "fragment" was

passed to the IUT's Remote Driver. The total lack of a

response datagram display is also an acceptable response.

Success: An ICMP Parameter Problem message, ICMP reassembly

timeout message, or nothing.

NOTICE: The receipt of an ICMP reassembly timeout message

(for a fragment with a non-zero offset) indicates that the

IUT does not conform to RFC792.  It is not grounds for the

denial of MIL-STD-1777 certification.

Failure: A reply datagram containing "ident".

Test 572 fragment with an enormous fragment offset

-Can the IUT deal with fragment with enormous fragment

offset;?

-Action:  Send a perfectly legal fragment to the IP

Implementation Under Test with an enormous fragment offset.

The IUT should either discard the fragment or create a state

vector to prepare to receive additional fragments.

If a state vector is created, its timer should expire and

the state vector should be discarded. In addition, the IUT

may send an ICMP reassembly timeout message to the IP

Tester.

-Verification:  If there is an IP Tester response datagram

display, it should contain "ICMP TIME EXCEEDED id=00572

REAS". It should not contain "ICMP PARAMETER PROBLEM

id=00572" because that would indicate that the IUT

considered the perfectly legal fragment defective.

The total lack of a response datagram display is also an

acceptable response.

Success: An ICMP reassembly timeout message, or nothing.

NOTICE: The receipt of an ICMP reassembly timeout message

(for a fragment with a non-zero offset) indicates that the

IUT does not conform to RFC792.  It is not grounds for the

denial of MIL-STD-1777 certification.

Failure: An ICMP Parameter Problem message.

Test 573 fragments spanning upper range of fragment offsets

-Can the IUT deal with upper range of fragment offsets?

-Action:  Send 79 perfectly legal fragments to the IP

Implementation Under Test spanning the upper range of

fragment offsets. For each fragment, the IUT should either

discard the fragment or create a state vector to prepare to

receive additional fragments.

If a state vector is created, its timer should expire before

the next fragment is sent to the IUT. The state vector

should be discarded. in addition, the IUT may send an ICMP

reassembly timeout message to the IP Tester.

-Verification: The IP Tester display of each possible

response datagram should contain "ICMP TIME EXCEEDED

id=00573 REAS". No IP Tester response datagram display

should contain "ICMP PARAMETER PROBLEM id=00573" because

that would indicate that the IUT considered a perfectly

legal fragment defective.

The total lack of a response datagram display for any

fragment is also an acceptable response for that fragment.

Success: An ICMP reassembly timeout message, or nothing.

NOTICE: The receipt of an ICMP reassembly timeout message

(for a fragment with a non-zero offset) indicates that the

IUT does not conform to RFC792. It is not grounds for the

denial of MIL-STD-1777 certification.

Failure: An ICMP Parameter Problem message.

Test 574 precedence mismatch

-Can the IUT detect a reassembly precedence mismatch?

-Action:  The IP Tester sends a fragment with precedence I

to the IP Implementation Under Test. This should cause the

IUT to create a state vector. The IP Tester will also send

another fragment with precedence 0 to the IP Implementation

Under.Test. The IUT should then send "an error datagram"

(presumably an ICMP Parameter Problem message) to the IP

Tester to indicate that a precedence mismatch was detected.

The current reassembly should be discarded. This is followed

by the three remaining fragments sent by the IP Tester to

the IUT. These are not sufficient to reassemble a datagram,

so the IUT should eventually discard the second reassembly.

The IUT may send an ICMP reassembly timeout message to the

IP Tester.

-Verification: The IP Tester display ought to contain the

string "ICMP PARAMETER PROBLEM id=00574". This ought to be

followed by the string "ICMP TIME EXCEEDED id=00574 REAS".

Neither is strictly required by section 4.2.3.

The total lack of a response datagram display is also an

acceptable response.

An IP Tester display containing "ident = 574 " indicates

that the IUT improperly reassembled the fragments and passed

the result to its Remote Driver.

Success: An optional ICMP Parameter Problem message followed

by an optional ICMP reassembly timeout message.

NOTICE: The receipt of an ICMP message (for a fragment with

a non-zero offset) indicates that the IUT does not conform

to RFC792.  It is not grounds for the denial of MIL-STD-1777

certification.

Failure: An IP Tester datagram display containing "ident =

574 ".

Test 575 security level mismatch

-Can the IUT detect a reassembly security level mismatch?

-Action:  The IP Tester sends a confidential fragment to the

IP Implementation Under Test. This should cause the IUT to

create a state vector. The IP Tester will also send an

unclassified fragment to the IP Implementation Under Test.

The IUT should then send "an error datagram" (presumably an

ICMP Parameter Problem message) to the IP Tester to indicate

that a security level mismatch was detected. The current

reassembly should be discarded. This is followed by the

three remaining fragments sent by the IP Tester to the IUT.

These are not sufficient to reassemble a datagram, so the

IUT should eventually discard the second reassembly. The IUT

may send an ICMP reassembly timeout message to the IP

Tester.

-Verification: The IP-Tester display ought to contain the

string "ICMP PARAMETER PROBLEM id=00575". This ought to be

followed by the string "ICMP TIME EXCEEDED id=00575 REAS".

Neither is strictly required by section 4.2.3.

The total lack of a response datagram display is also an

acceptable response.

An IP Tester display containing "ident = 575 " indicates

that the IUT improperly reassembled the fragments and passed

the result to its Remote Driver.

Success: An optional ICMP Parameter Problem message followed

by an optional ICMP reassembly timeout message.

NOTICE: The receipt of an ICMP message (for a fragment with

a non-zero offset) indicates that the IUT does not conform

to RFC792.  It is not grounds for the denial of MIL-STD-1777

certification.

Failure: An IP Tester datagram display containing "ident =

575 ".

Test 576 security level mismatch

-Can the IUT detect a reassembly security level mismatch?

-Action:  The IP Tester sends a secret fragment to the IP

Implementation Under Test. This should cause the IUT to

create a state vector. The IP Tester will also send a top

secret fragment to the IP Implementation Under Test. The IUT

should then send "an error datagram" (presumably an ICMP

Parameter Problem message) to the IP Tester to indicate that

a security level mismatch was detected. The current

reassembly should be discarded. This is followed by the

three remaining fragments sent by the IP Tester to the IUT.

These are not sufficient to reassemble a datagram, so the

IUT should eventually discard the second reassembly. The IUT

may send an ICMP reassembly timeout message to the IP

Tester.

-Verification: The IP Tester display ought to contain the

string "ICMP PARAMETER PROBLEM id=00576". This ought to be

followed by the string "ICMP TIME EXCEEDED id=00576 REAS".

Neither is strictly required by section 4.2.3.

The total lack of a response datagram display is also an

acceptable response.

An IP Tester display containing "ident = 576 " indicates

that the IUT improperly reassembled the fragments and passed

the result to its Remote Driver.

Success: An optional ICMP Parameter Problem message followed

by an optional ICMP reassembly timeout message.

NOTICE: The receipt of an ICMP message (for a fragment with

a non-zero offset) indicates that the IUT does not conform

to RFC792. It is not grounds for the denial of MIL-STD-1777

certification.

Failure: An IP Tester datagram display containing "ident

576 ".

Test 577 security level mismatch

-Can the IUT detect a reassembly security level mismatch?

-Action:  The IP Tester sends an unclassified fragment to

the IP Implementation Under Test. This should cause the IUT

to create a state vector. The IP Tester will also send a

secret fragment to the IP Implementation Under Test. The IUT

should then send "an error datagram" (presumably an ICMP

Parameter Problem message) to the IP Tester to indicate that

a security level mismatch was detected. The current

reassembly should be discarded. This is followed by the

three remaining fragments sent by the IP Tester to the IUT.

These are not sufficient to reassemble a datagram, so the

IUT should eventually discard the second reassembly. The IUT

may send an ICMP reassembly timeout message to the IP

Tester.

-Verification: The IP Tester display ought to contain the

string "ICMP PARAMETER PROBLEM id=00577". This ought to be

followed by the string "ICMP TIME EXCEEDED id=00577 REAS".

Neither is strictly required by section 4.2.3.

The total lack of a response datagram display is also an

acceptable response.

An IP Tester display containing "ident = 577 " indicates

that the IUT improperly reassembled the fragments and passed

the result to its Remote Driver.

Success: An optional ICMP Parameter Problem message followed

by an optional ICMP reassembly timeout message.

NOTICE: The receipt of an ICMP message (for a fragment with

a non-zero offsat) indicates that the IUT does not conform

to RFC792.  It is not grounds for the denial of MIL-STD-1777

certification.

Failure: An IP Tester datagram display containing "ident =

577 ".

Test 578 first compartmentation byte mismatch

-Can the IUT detect a reassembly compartmentation mismatch?

-Action:  The IP Tester sends a fragment to the IP

Implementation Under Test. This should cause the IUT to

create a state vector. The IP Tester will also send a

fragment to the IP Implementation Under Test whose first

compartmentation byte differs from the first fragment's. The

IUT probably should then send "an error datagram"

(presumably an ICMP Parameter Problem message) to the IP

Tester to indicate that a security mismatch was detected.

The current reassembly probably should be discarded. This is

followed by the three remaining fragments sent by the IP

Tester to the IUT. These are not sufficient to reassemble a

datagram, so the IUT should eventually discard the second

reassembly. The IUT may send an ICMP reassembly timeout

message to the IP Tester.

Because section 9.4.6.3.9.h refers to the fragments'

security levels, the IUT may reassemble the five fragments

sent to it by the IP Tester and pass the result to its

Remote Driver. The IUT's Remote Driver should then request

the IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

Verification: The IP Tester display ought to contain the

string "ICMP PARAMETER PROBLEM id=00578". This ought to be

followed by the string "ICMP TIME EXCEEDED id=00578 REAS".

Neither is strictly required by section 4.2.3.

The total lack of a response datagram display is also an

acceptable response.

If the IUT chooses to reassemble the fragments, the IP

Tester reply datagram display should display a 22-byte data

field containing "ident = 578 datalen = ".

Success: An optional ICMP Parameter Problem message followed

by an optional ICMP reassembly timeout message. Alternately,

a reply datagram containing "ident = 578 datalen = ".

NOTICE: The receipt of an ICMP message (for a fragment with

a non-zero offset) indicates that the IUT does not conform

to RFC792.  It is not grounds for the denial of MIL-STD-1777

certification.

Failure: A datagram other than an ICMP Parameter Problem

message, an ICMP reassembly timeout message, or the reply

datagram containing "ident = 578 datalen = ". An ICMP

message followed by the reply datagram containing "ident =

578 datalen = " is also a failure.

Test 579 second compartmentation byte mismatch

-Can the IUT detect a reassembly compartmentation mismatch?

-Action:  The IP Tester sends a fragment to the IP

Implementation Under Test. This should cause the IUT to

create a state vector. The IP Tester will also send a

fragment to the IP Implementation Under Test whose second

compartmentation byte differs from the first fragment's. The

IUT probably should then send "an error datagram"

(presumably an ICMP Parameter Problem message) to the IP

Tester to indicate that a security mismatch was detected.

The current reassembly probably should be discarded. This is

followed by the three remaining fragments sent by the IP

Tester to the IUT. These are not sufficient to reassemble a

datagram, so the IUT should eventually discard the second

reassembly. The IUT may send an ICMP reassembly timeout

message to the IP Tester.

Because section 9.4.6.3.9.h refers to the fragments'

security levels, the IUT may reassemble the five fragments

sent to it by the IP Tester and pass the result to its

Remote Driver. The IUT's Remote Driver should then request

the IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

-Verification: The IP Tester display ought to contain the

string "ICMP PARAMETER PROBLEM id=00579". This ought to be

followed by the string "ICMP TIME EXCEEDED id=00579 REAS".

Neither is strictly required by section 4.2.3.

The total lack of a response datagram display is also an

acceptable response.

If the IUT chooses to reassemble the fragments, the IP

Tester reply datagram display should display a 21-byte data

field containing "ident = 579 datalen =".

Success: An optional ICMP Parameter Problem message followed

by an optional ICMP reassembly timeout message. Alternately,

a reply datagram containing "ident = 579 datalen =".

NOTICE: The receipt of an ICMP message (for a fragment with

a non-zero offset) indicates that the IUT does not conform

to RFC792.  It is not grounds for the denial of MIL-STD-1777

certification.

Failure: A datagram other than an ICMP Parameter Problem

message, an ICMP reassembly timeout message, or the reply

datagram containing "ident = 579 datalen =". An ICMP message

followed by the reply datagram containing "ident = 579

datalen =" is also a failure.

Test 580 first handling byte mismatch

-Can the IUT detect a reassembly handling mismatch?

-Action:  The IP Tester sends a fragment to the IP

Implementation Under Test. This should cause the IUT to

create a state vector. The IP Tester will also send a

fragment to the IP Implementation Under Test whose first

handling byte differs from the first fragment's. The IUT

probably should then send "an error datagram" (presumably an

ICMP Parameter Problem message) to the IP Tester to indicate

that a security mismatch was detected. The current

reassembly probably should be discarded. This is followed by

the three remaining fragments sent by the IP Tester to the

IUT. These are not sufficient to reassemble a datagram, so

the IUT should eventually discard the second reassembly. The

IUT may send an ICMP reassembly timeout message to the IP

Tester.

Because section 9.4.6.3.9.h refers to the fragments'

security- levels, the IUT may reassemble the five fragments

sent to it by the IP Tester and pass the result to its

Remote Driver. The IUT's Remote Driver should then request

the IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

-Verification: The IP Tester display ought to contain the

string "ICMP PARAMETER PROBLEM id=00580". This ought to be

followed by the string "ICMP TIME EXCEEDED id=00580 REAS".

Neither is strictly required by section 4.2.3.

The total lack of a response datagram display is also an

acceptable response.

If the IUT chooses to reassemble the fragments, the IP

Tester reply datagram display should display a 20-byte data

field containing "ident = 580 datalen ".

Success: An optional ICMP Parameter Problem message followed

by an optional ICMP reassembly timeout message. Alternately,

a reply datagram containing "ident = 580 datalen ".

NOTICE: The receipt of an ICMP message (for a fragment with

a non-zero offset) indicates that the IUT does not conform

to RFC792.  It is not grounds for the denial of MIL-STD-1777

certification.

Failure: A datagram other than an ICMP Parameter Problem

message, an ICMP reassembly timeout message, or the reply

datagram containing "ident = 580 datalen ". An ICMP;message

followed by the reply datagram containing "ident = 580

datalen " is also a failure.

Test 581 second handling byte mismatch

-Can the IUT detect a reassembly handling mismatch?

-Action:  The IP Tester sends a fragment to the IP

Implementation Under Test. This should cause the IUT to

create a state vector. The IP Tester will also send a

fragment to the IP Implementation Under Test whose second

handling byte differs from the first fragment's. The IUT

probably should then send "an error datagram" (presumably an

ICMP Parameter Problem message) to the IP Tester to indicate

thst a security mismatch was detected. The current

reassembly probably should be discarded. This is followed by

the three remaining fragments sent by the IP Tester to the

IUT. These are not sufficient to reassemble a datagram, so

the IUT should eventually discard the second reassembly. The

IUT may send an ICMP reassembly timeout message to the IP

Tester.

Because section 9.4.6.3.9.h refers to the fragments'

security levels, the IUT may reassemble the five fragments

sent to it by the IP Tester and pass the result to its

Remote Driver. The IUT's Remote Driver should then request

the IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

-Verification: The IP Tester display ought to contain the

string "ICMP PARAMETER PROBLEM id=00581". This ought to be

followed by the string "ICMP TIME EXCEEDED id=00581 REAS".

Neither is strictly required by section 4.2.3.

The total lack of a response datagram display is also an

acceptable response.

If the IUT chooses to reassemble the fragments, the IP

Tester reply datagram display should display a 19-byte data

field containing "ident = 581 datalen".

Success: An optional ICMP Parameter Problem message followed

by an optional ICMP reassembly timeout message. Alternately,

a reply datagram containing "ident = 581 datalen".

NOTICE: The receipt of an ICMP message (for a fragment with

a non-zero offset) indicates that the IUT does not conform

to RFC792.  It is not grounds for the denial of MIL-STD-1777

certification.

Failure: A datagram other than an lCMP Parameter Problem

message, an ICMP reassembly timeout message, or the reply

datagram containing "ident = 581 datalen". An ICMP message

followed by the reply datagram containing "ident = 581

datalen" is also a failure.

Test 582 first transmission control code byte mismatch

-Can the IUT detect a reassembly TCC mismatch?

-Action:  The IP Tester sends a fragment to the IP

Implementation Under Test. This should cause the IUT to

create a state vector. The IP Tester will also send a

fragment to the IP Implementation Under Test whose first

transmission control code byte differs from the first

fragments.  The IUT probably should then send "an error

datagram" (presumably an ICMP Parameter Problem message) to

the IP Tester to indicate that a security mismatch was

detected. The current reassembly probably should be

discarded. This is followed by the three remaining fragments

sent by the IP Tester to the IUT. These are not sufficient

to reassemble a datagram, so the IUT should eventually

discard the second reassembly. The IUT may send an ICMP

reassembly timeout message to the IP Tester.

Because section 9.4.6.3.9.h refers to the fragments'

security levels, the IUT may reassemble the five fragments

sent to it by the IP Tester and pass the result to its

Remote Driver. The IUT's Remote Driver should then request

the IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

-Verification: The IP Tester display ought to contain the

string "ICMP PARAMETER PROBLEM id=00582". This ought to be

followed by the string "ICMP TIME EXCEEDED id=00582 REAS".

Neither is strictly required by section 4.2.3.

The total lack of a response datagram display is also an

acceptable response.

If the IUT chooses to reassemble the fragments, the IP

Tester reply datagram display should display a 18-byte data

field containing "ident = 582 datale".

Success: An optional ICMP Parameter Problem message followed

by an optional ICMP reassembly timeout message. Alternately,

a reply datagram containing "ident = 582 datale".

NOTICE: The receipt of an ICMP message (for a fragment with

a non-zero offset) indicates that the IUT does not conform

to RFC792.  It is not grounds for the denial of MIL-STD-1777

certification.

Failure: A datagram other than an ICMP Parameter Problem

message, an ICMP reassembly timeout message, or the reply

datagram containing "ident = 582 datale". An ICMP message

followed by the reply datagram containing "ident = 582

datale" is also a failure.

Test 583 second transmission control code byte mismatch

-Can the IUT detect a reassembly TCC mismatch?

-Action:  The IP Tester sends a fragment to the IP

Implementation Under Test. This should cause the IUT to

create a state vector. The IP Tester will also send a

fragment to the IP Implementation Under Test whose second

transmission control code byte differs from the first

fragment's. The IUT probably should then send "an error

datagram" (presumably an ICMP Parameter Problem message) to

the IP Tester to indicate that a security mismatch was

detected. The current reassembly probably should be

discarded. This is followed by the three remaining fragments

sent by the IP Tester to the IUT. These are not sufficient

to reassemble a datagram, so the IUT should eventually

discard the second reassembly. The IUT may send an ICMP

reassembly timeout message to the IP Tester.

Because section 9.4.6.3.9.h refers to the fragments,

security levels, the IUT may reassemble the five fragments

sent to it by the IP Tester and pass the result to its

Remote Driver. The IUT's Remote Driver should then request

the IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

-Verification: The IP Tester display ought to contain the

string "ICMP PARAMETER PROBLEM id=00583". This ought to be

followed by the string "ICMP TIME EXCEEDED id=00583 REAS".

Neither is strictly required by section 4.2.3.

The total lack of a response datagram display is also an

acceptable response.

If the IUT chooses to reassemble the fragments, the IP

Tester reply datagram display should display a 17-byte data

field containing "ident = 583 datal".

Success: An optional ICMP Parameter Problem message followed

by an optional ICMP reassembly timeout message. Alternately,

a reply datagram containing "ident = 583 datal".

NOTICE: The receipt of an ICMP message (for a fragment with

a non-zero offset) indicates that the IUT does not conform

to RFC792.  It is not grounds for the denial of MIL-STD-1777

certification.

Failure: A datagram other than an ICMP Parameter Problem

message, an ICMP reassembly timeout message, or the reply

datagram containing "ident = 583 datal". An ICMP message

followed by the reply datagram containing "ident = 583

datal" is also a failure.

Test 584 third transmission control code byte mismatch

-Can the IUT detect a reassembly TCC mismatch?

-Action:  The IP Tester sends a fragment to the IP

Implementation Under Test. This should cause the IUT to

create a state vector. The IP Tester will also send a

fragment to the IP Implementation Under Test whose third

transmission control code byte differs from the first

fragment's. The IUT probably should then send "an error

datagram" (presumably an ICMP Parameter Problem message) to

the IP Tester to indicate that a security mismatch was

detected. The current reassembly probably should be

discarded. This is followed by the three remaining fragments

sent by the IP Tester to the IUT. These are not sufficient

to reassemble a datagram, so the IUT should eventually

discard the second reassembly. The IUT may send an ICMP

reassembly timeout message to the IP Tester.

Because section 9.4.6.3.9.h refers to the fragments,

security levels, the IUT may reassemble the five fragments

sent to it by the IP Tester and pass the result to its

Remote Driver. The IUT's Remote Driver should then request

the IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

-Verification: The IP Tester display ought to contain the

string "ICMP PARAMETER PROBLEM id=00584". This ought to be

followed by the string "ICMP TIME EXCEEDED id=00584 REAS".

Neither is strictly required by section 4.2.3.

The total lack of a response datagram display is also an

acceptable response.

If the IUT chooses to reassemble the fragments, the IP

Tester reply datagram display should display a 16-byte data

field containing "ident = 584 data".

Success: An optional ICMP Parameter Problem message followed

by an optional ICMP reassembly timeout message. Alternately,

a reply datagram containing "ident = 584 data".

NOTICE: The receipt of an ICMP message (for a fragment with

a non-zero offset) indicates that the IUT does not conform

to RFC792.  It is not grounds for the denial of MIL-STD-1777

certification.

Failure: A datagram other than an ICMP Parameter Problem

message, an ICMP reassembly timeout message, or the reply

datagram containing "ident = 584 data". An ICMP message

followed by the reply datagram containing "ident = 584 data"

is also a failure.

Test 585 security level mismatch

-Can the IUT detect a reassembly security level mismatch?

-Action:  The IP Tester sends a confidential fragment to the

IP Implementation Under Test. This should cause the IUT to

create a state vector. The IP Tester will also send a

fragment without a security option to the IP Implementation

Under Test. The IUT should then send "an error datagram"

(presumably an ICMP Parameter Problem message) to the IP

Tester to indicate that a security level mismatch was

detected. The current reassembly should be discarded. This

is followed by the three remaining fragments sent by the IP

Tester to the IUT. These are not sufficient to reassemble

datagram, so the IUT should eventually discard the second

reassembly. The IUT may send an ICMP reassembly timeout

message to the IP Tester.

-Verification: The IP Tester display ought to contain the

string "ICMP PARAMETER PROBLEM id=00585". This ought to be

followed by the string "ICMP TIME EXCEEDED id=00585 REAS".

Neither is strictly required by section 4.2.3.

The total lack of a response datagram display is also an

acceptable response.

An IP Tester display containing "ident = 585 " indicates

that the IUT improperly reassembled the fragments and passed

the result to its Remote Driver.

Success: An optional ICMP Parameter Problem message followed

by an optional ICMP reassembly timeout message.

NOTICE: The receipt of an ICMP message (for a fragment with

a non-zero offset) indicates that the IUT does not conform

to RFC792.  It is not grounds for the denial of MIL-STD-1777

certification.

Failure: An IP Tester datagram display containing "ident =

585 ".

Scenario File OpBdRouBotTest: "Bottom-up Bad Routing Option"

datagrams

2100      truncated loose source and record route option

2101      add one to option length

2102      add another byte to option

2103      remove byte from option

2104      option length of 0

2105      option length of 377

2106      option length of 353

2107      option length of 177

2108      option length of 203

2109      option length of 207

2110      add one to option pointer

2111      subtract one from option pointer

2112      option pointer of 300

2113      option pointer of 174

2114      add twelve to option pointer and option-length

2115      option pointer just past maximum legal value

2116      option pointer just past larger maximum legal value

2117      duplicated option

2118      two different options of same type

2119      normal datagram

2120      truncated strict source and record route option

2121      add one to option length

2122      add another byte to option

2123      remove byte from option

2124      option length of 0

2125      option length of 377

2126      option length of 353

2127      option length of 177

2128      option length of 203

2129      option length of 207

2130      add one to option pointer

2131      subtract one from option pointer

2132      option pointer of 300

2133      option pointer of 174

2134      add twelve to option pointer and option length

2135      option pointer just past maximum legal value

2136      option pointer just past larger maximum legal value

2137      duplicated option

2138      two different options of same type

2139      normal datagram

2140      bad option followed by good option

2141      bad option surrounded by good options

2142      normal datagram

-----------------------------------

Test 2100 truncated loose source and record route option

-Can the IUT interpret a truncated option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. The first two bytes are no

operation options (to permit the truncation of the last

option). Because the address in the option isn't all there,

the IUT probably should send an ICMP Parameter Problem

message to the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02lOO". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2100 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2100 ", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2119).

Test 2101 loose source and record route option length bumped by

one

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. The first byte is a no

operation option to distinguish this test from Test 2102.

This is followed by a loose source and record route option

whose option length has been bumped by one. The IUT may send

an ICMP Parameter Problem message to the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02101". The alternate ICMP Unreachable

message display must contain the string: "ICMP DESTINATION

UNREACHABLE id=02101 HOST".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2102 loose source and record route option one byte too long

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This consists of a loose

source and record route option that is one byte too long.

The IUT may send an ICMP Parameter Problem message to the IP

Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester.

The IUT should always discard the datagram.

-Verification: If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02102". The alternate ICMP Unreachable

message display must contain the string: "ICMP DESTINATION

UNREACHABLE id=02102 HOST".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2103 loose source and record route option one byte too short

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This consists of a loose

source and record route option that is one byte too short

followed by an end of option list option and one byte of

padding. The IUT probably should send an ICMP Parameter

Problem message to the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02103". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2103 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2103 ", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2119).

Test 2104 loose source and record route option length of 0

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a loose

source and record route option with an option length of 0.

The IUT should discard the datagram. The IUT may send an

ICMP Parameter Problem message to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02104".A display of an ICMP message

should indicate that it came from the IUT by containing the

string "S00701600002".

Success: An optional ICMP Parameter Problem message.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2119).

Test 2105 loose source and record route option length of 377

octal

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a loose

source and record route option with an option length of 377

octal (neRative 1). The IUT may send an ICMP Parameter

Problem message to the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02105". The alternate ICMP Unreachable

message display must contain one of the three strings:  "ICMP

DESTINATION UNREACHABLE id=02105 HOST", "ICMP DESTINATION

UNREACHABLE id=02105 NETW", or "ICMP DESTINATION UNREACHABLE

id=02105 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2106 loose source and record route option length of 353

octal

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a loose

source and record route option with an option length of 353

octal. The IUT may send an ICMP Parameter Problem message to

the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02106". The alternate ICMP Unreachable

message display must contain one of the three strings: "ICMP

DESTINATION UNREACHABLE id=02106 HOST", "ICMP DESTINATION

UNREACHABLE id=02106 NETW", or "ICMP DESTINATION UNREACHABLE

id=02106 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2107 loose source and record route option length of 177

octal

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a loose

source and record route option with an option length of 177

octal. The IUT may send an ICMP Parameter Problem message to

the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02107". The alternate ICMP Unreachable

message display must contain one of the three strings: "ICMP

DESTINATION UNREACHABLE id=02107 HOST", "ICMP DESTINATION

UNREACHABLE id=02107 NETW", or "ICMP DESTINATION UNREACHABLE

id=02107 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2108 loose source and record route option length of 203

octal

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a loose

source and record route option with an option length of 203

octal. The IUT may send an ICMP Parameter Problem message to

the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02108". The alternate ICMP Unreachable

message display must contain one of the three strings:  "ICMP

DESTINATION UNREACHABLE id=02108 HOST", "ICMP DESTINATION

UNREACHABLE id=02108 NETW", or "ICMP DESTINATION UNREACHABLE

id=02108 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2109 loose source and record route option length of 207

octal

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a loose

source and record route option with an option length of 207

octal (sign bit set). The IUT may send an ICMP Parameter

Problem message to the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02109". The alternate ICMP Unreachable

message display must contain one of the three strings: "ICMP

DESTINATION UNREACHABLE id=02109 HOST", "ICMP DESTINATION

UNREACHABLE id=02109 NETW", or "ICMP DESTINATION UNREACHABLE

id=02109 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2110 loose source and record route option pointer bumped by

1

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This consists of a loose

source and record route option whose pointer has been bumped

by 1. The IUT may send an ICMP Parameter Problem message to

the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02llO". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2110 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2110 ", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2119).

Test 2111 loose source and record route option with pointer 1

short

-Can the IUT interpret a defective option pointer?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a loose

source and record route option with an option pointer one

short of the legal minimum. The IUT may send an ICMP

Parameter Problem message to the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02111". The alternate ICMP Unreachable

message display must contain one of the three strings: "ICMP

DESTINATION UNREACHABLE id=02111 HOST", "ICMP DESTINATION

UNREACHABLE id=02111 NETW", or "ICMP DESTINATION UNREACHABLE

id=02111 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2112 loose source and record route option pointer of 300

octal

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a loose

source and record route option whose pointer has been set to

300 octal. The IUT may send an ICMP Parameter Problem

message to the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02112". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2112 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2112", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2119).

Test 2113 loose source and record route option pointer of 174

octal

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a loose

source and record route option whose pointer has been set to

174 octal. The IUT may send an ICMP Parameter Problem

message to the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification: If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02113". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2113 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2113", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2119).

Test 2114 loose source and record route option with bad option

length and option pointer

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a loose

source and record route option whose option length and

option pointer have been bumped by 12. The IUT may send an

ICMP Parameter Problem message to the IP Tester and discard

the datagram.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02114". The alternate ICMP Unreachable

message display must contain one of the three strings:  "ICMP

DESTINATION UNREACHABLE id=02114 HOST", "ICMP DESTINATION

UNREACHABLE id=02114 NETW", or "ICMP DESTINATION UNREACHABLE

id=02114 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2115 loose source and record route option with bad option

pointer

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a single

address loose source and record route option whose pointer

has been set to just exceed its maximum legal value. The IUT

may send an ICMP Parameter Problem message to the IP Tester

and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02115". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2115 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2115 ", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2119).

Test 2116 loose source and record route option with bad option

pointer

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 12-byte option field. This contains a two

address loose source and record route option whose pointer

has been set to just exceed its maximum legal value. The IUT

may send an ICMP Parameter Problem message to the IP Tester

and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification: If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02116". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2116 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2116 ", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2119).

Test 2117 loose source and record route option duplicated

-Can the IUT interpret a duplicated option?

-Action:  Send a datagram to the IP Implementation Under

Test whose option field contains two identical loose source

and record route options. The IUT may send an ICMP Parameter

Problem message to the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02117". The alternate ICMP Unreachable

message display must contain the string: "ICMP DESTINATION

UNREACHABLE id=02117 HOST".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2118 two loose source and record route options

-Can the IUT detect two options of same type ?

-Action:  Send a datagram to the IP Implementation Under

Test whose option field contains two loose source and record

route options. The IUT may send an ICMP Parameter Problem

message to the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02118". The alternate ICMP Unreachable

message display must contain the string: "ICMP DESTINATION

UNREACHABLE id=02118 HOST".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2119 ordinary datagram after bad LSRR tests

-Can the IUT send datagram after processing nastygrams?

-Action:  Send a datagram to the IP Implementation Under

Test. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

-Verification: The IP Tester will display the reply datagram

that must contain "ident = 2119 " in its 13-byte data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram. The actual failure probably

occurred during Tests 2100-2116.

Test 2120 truncated strict source and record route option

-Can the IUT interpret a truncated option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. The first two bytes are no

operation options (to permit the truncation of the last

option). Because the address in the option isn't all there,

the IUT probably should send an ICMP Parameter Problem

message to the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02120". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2120 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2120 ", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2139).

Test 2121 strict source and record route option length bumped by

one

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. The first byte is a no

operation option to distinguish this test from Test 2122.

This is followed by a strict source and record route option

whose option length has been bumped by one. The IUT may send

an ICMP Parameter Problem message to the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02121". The alternate ICMP Unreachable

message display must contain the string: "ICMP DESTINATION

UNREACHABLE id=02121 HOST".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2122 strict source and record route option one byte too long

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This consists of a strict

source and record route option that is one byte too long.

The IUT may send an ICMP Parameter Problem message to the IP

Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02122". The alternate ICMP Unreachable

message display must contain the string: "ICMP DESTINATION

UNREACHABLE id=02122 HOST".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2123 strict source and record route option one byte too

short

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This consists of a strict

source and record route option that is one byte too short

followed by an end of option list option and one byte of

padding. The IUT probably should send an ICMP Parameter

Problem message to the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02123". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2123 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2123 ", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2139).

Test 2124 strict source and record route option length of 0

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a strict

source and record route option with an option length of 0.

The IUT should discard the datagram. The IUT may send an

ICMP Parameter Problem message to the IP Tester.

-Verification: If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02124".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem message.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2139).

Test 2125 strict source and record route option length of 377

octal

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a strict

source and record route option with an option length of 377

octal (negative 1). The IUT may send an ICMP Parameter

Problem message to the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02125". The alternate ICMP Unreachable

message display must contain one of the three strings:  "ICMP

OESTINATION UNREACHABLE id=02125 HOST", "ICMP DESTINATION

UNREACHABLE id=02125 NETW", or "ICMP DESTINATION UNREACHABLE

id=02125 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2126 strict source and record route option length of 353

octal

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a strict

source and record route option with an option length of 353

octal. The IUT may send an ICMP Parameter Problem message to

the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02126". The alternate ICMP Unreachable

message display must contain one of the three strings: "ICMP

DESTINATION UNREACHABLE id=02126 HOST", "ICMP DESTINATION

UNREACHABLE id=02126 NETW", or "ICMP DESTINATION UNREACHABLE

id=02126 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by contaiining the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2127 strict source and record route option length of 177

octal

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a strict

source and record route option with an option length of 177

octal. The IUT may send an ICMP Parameter Problem message to

the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification: If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02127". The alternate ICMP Unreachable

message display must contain one of the three strings: "ICMP

DESTINATION UNREACHABLE id=02127 HOST", "ICMP DESTINATION

UNREACHABLE id=02127 NETW", or "ICMP DESTINATION UNREACHABLE

id=02127 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2128 strict source and record route option length of 203

octal

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a strict

source and record route option with an option length of 203

octal. The IUT may send an ICMP Parameter Problem message to

the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02128". The alternate ICMP Unreachable

message display must contain one of the three strings: "ICMP

DESTINATION UNREACHABLE id=02128 HOST", "ICMP DESTINATION

UNREACHABLE id=02128 NETW", or "ICMP DESTINATION UNREACHABLE

id=02128 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2129 strict source and record route option length of 207

octal

-Can the IUT interpret a defective option length?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a strict

source and record route option with an option length of 207

octal (sign bit set). The IUT may send an ICMP Parameter

Problem message to the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02129". The alternate ICMP Unreachable

message display must contain one of the three strings: "ICMP

DESTINATION UNREACHABLE id=02129 HOST", "ICMP DESTINATION

UNREACHABLE id=02129 NETW", or "ICMP DESTINATION UNREACHABLE

id=02129 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2130 strict source and record route option pointer bumped by

1

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This consists of a strict

source and record route option whose pointer has been bumped

by 1. The IUT may send an ICMP Parameter Problem message to

the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification: If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02130". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2130 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2130 ", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2139).

Test 2131 strict source and record route option with pointer one

short

-Can the IUT interpret a defective option pointer?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a strict

source and record route option with an option pointer one

short of the legal minimum. The IUT may send an ICMP

Parameter Problem message to the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02131". The alternate ICMP Unreachable

message display must contain one of the three strings: "ICMP

DESTINATION UNREACHABLE id=02131 HOST", "ICMP DESTINATION

UNREACHABLE id=02131 NETW", or "ICMP DESTINATION UNREACHABLE

id=02131 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2132 strict source and record route option pointer of 300

octal

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a strict

source and record route option whose pointer has been set to

300 octal. The IUT may send an ICMP Parameter Problem

message to the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02132". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2132 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2132 ", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2139).

Test 2133 strict source and record route option pointer of 174

octal

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a strict

source and record route option whose pointer has been set to

174 octal. The IUT may send an ICMP Parameter Problem

message to the IP Tester and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02133". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2133 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2133 ", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2139).

Test 2134 strict source and record route option with bad option

length and option pointer

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a strict

source and record route option whose option length and

option pointer have been bumped by 12. The IUT may send an

ICMP Parameter Problem message to the IP Tester and discard

the datagram.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester. Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message. Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02134". The alternate ICMP Unreachable

message display must contain one of the three strings:  "ICMP

DESTINATION UNREACHABLE id=02134 HOST", "ICMP DESTINATION

UNREACHABLE id=02134 NETW", or "ICMP DESTINATION UNREACHABLE

id=02134 ROUT".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success:  An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2135 strict source and record route option with bad option

pointer

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 8-byte option field. This contains a single

address strict source and record route option whose pointer

has been set to just exceed its maximum legal value. The IUT-

may send an ICMP Parameter Problem message to the IP Tester

and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02135". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2135 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2135 ", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2139).

Test 2136 strict source and record route option with bad option

pointer

-Can the IUT interpret a defective option?

-Action:  Send a datagram to the IP Implementation Under

Test with an 12-byte option field. This contains a two

address strict source and record route option whose pointer

has been set to just exceed its maximum legal value. The IUT

may send an ICMP Parameter Problem message to the IP Tester

and discard the datagram.

It is possible that the IUT will accept the datagram and

pass it to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

-Verification:  If there is a response datagram displayed by

the IP Test-r, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02136". However, if the IUT chooses to

pass the datagram to its Remote Driver, the reply datagram

must contain "ident = 2136 " in its 13-byte data field.

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An ICMP Parameter Problem message, reply datagram

containing "ident = 2136 ", or nothing.

Failure: A failure to pass the next test performed after

this test that clearly requires a positive response from the

IUT (Test 2139).

Test 2137 strict source and record route option duplicated

-Can the IUT interpret a duplicated option?

-Action:  Send a datagram to the IP Implementation Under

Test whose option field contains two identical strict source

and record route options. The IUT may send an ICMP Parameter

Problem message to the IP Tester.

The IUT may attempt to forward the datagram instead. Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02137". The alternate ICMP Unreachable

message display must contain the string: "ICMP DESTINATION

UNREACHABLE id=02137 HOST".

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002".

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2138 two strict source and record route options

-Can the IUT detect two options of same type?

-Action:  Send a datagram to the IP Implementation Under

Test whose option field contains two strict source and

record route options.  The IUT may send an ICMP Parameter

Problem message to the IP Tester.

The IUT may attempt to forward the datagram instead.  Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID=02138."  The alternate ICMP Unreachable

message display must contain the string:  "ICMP DESTINATION

UNREACHABLE id-02138 HOST."

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002."

Success:  An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure:  An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2139 ordinary datagram after bad SSRR Tests

-Can the IUT send datagram after processing nastygrams?

-Action:  Send a datagram to the IP Implementation Under

Test.  This datagram should be passed by the IUT to its

Remote Driver.  The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

-Verification:  The IP Tester will display the reply datagram

that must contain "ident = 2139" in its 13-byte data field.

Success:  Display of reply datagram requested to be sent.

Failure:  Any response other than the display of the

requested reply datagram.  The actual failure probably

occurred during Tests 2120-2136.

Test 2140 bad loose source and record route and good stream ID

option

-Can the IUT remember detecting a bad option?

-Action:  Send a datagram to the IP Implementation Under

Test whose option field contains a bad loose source and

record route option followed by a good stream identification

option.  The IUT may send an ICMP Parameter Problem message

to the IP Tester.

The IUT may attempt to forward the datagram instead.  Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester.  Section 4.3.l Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message.  Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "CIMP

PARAMETER PROBLEM ID=02140."  The alternate ICMP Unreachable

message display must contain one of the three strings:  "ICMP

DESTINATION UNREACHABLE id=02140 HOST," ICMP DESTINATION

UNREACHABLE id=02140 NETW," or " ICMP DESTINATION UNREACHABLE

id=02140 ROUT."

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002."

Success:  An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure:  An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

messagae.

Test 2141 bad loose source and record route and two good security

options

-Can the IUT remember detecting a bad option?

-Action:  Send a datagram to the IP Implementation Under

Test whose option field contains a good unclassified

security option followed by a bad loose source and record

route option followed by a good top secret security option.

The IUT may send an ICMP Parameter Problem message to the IP

Tester.

The IUT may attempt to forward the datagram instead.  Section

9.4.6.1.2 (Host) IPs may send an ICMP Unreachable Host

message to the IP Tester.  Section 4.3.1 Gateway IPs may send

an ICMP Unreachable Net or ICMP Unreachable Source Route

Failed message.  Unless instructed otherwise, the lab will

consider this behavior conformant.

The IUT should always discard the datagram.

-Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain the string "ICMP

PARAMETER PROBLEM ID+02141."  The alternate ICMP Unreachable

message display must contain one of the three strings:  "ICMP

DESTINATION UNREACHABLE id=02141 HOST," "ICMP DESTINATION

UNREACHABLE id=02141 NETW," or "ICMP DESTINATION UNREACHABLE

id-02141 ROUT."

A display of an ICMP message should indicate that it came

from the IUT by containing the string "S00701600002."

Success: An optional ICMP Parameter Problem or ICMP

Destination Unreachable message.

Failure:  An IP Tester display of a datagram other than an

ICMP Parameter Problem or ICMP Destination Unreachable

message.

Test 2142 ordinary datagram after bad source routing tests

-Can the IUT send datagram after processing nastygrams?

-Action:  Send a datagram to the IP Implementation Under

Test.  This datagram should be passed by the IUT to its

Remote Driver.  The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

-Verification:  The IP Tester will display the reply datagram

that must contain "ident = 2142" in its 13-byte data field.

Success:  Display of reply datagram requested to be sent.

Failure:  Any response other than the display of the

requested reply datagram.  Failure of this test is the most

critical failure in the scenario.

Scenario File OpBdRouTopTest:  "Top-down Bad Routing Option"

datagram

4100	truncated loose source and record route option

4101	add one to option length

4102	add another byte to option

4103	remove byte from option

4104	option length of 0

4105	option length of 377

4106	option length of 353

4107	option length of 177

4108	option length of 203

4109	option length of 207

4110	add one to option pointer

4111	subtract one from option pointer

4112	option pointer of 300

4113	option pointer of 174

4114	add twelve to option pointer and option length

4115	option pointer just past maximum legal value

4116	option pointer just past larger maximum legal value

4117	duplicated option

4118	two different options of same type

4119	normal datagram

4120	truncated strict source and record route option

4121	add one to option length

4122	add another byte to option

4123	remove byte from option

4124	option length of 0

4125	option length of 377

4126	option length of 353

4127	option length of 177

4128	option length of 203

4129	option length of 207

4130	add one to option pointer

4131	subtract one from option pointer

4132	option pointer of 300

4133	option pointer of 174

4134	add twelve to option pointer and option length

4135	option pointer just past maximum legal value

4136	option pointer just past larger maximum legal value

4137	duplicated option

4138	two different options of same type

4139	normal datagram

4140	bad option followed by good option

4141	bad option surrounded by good options

4142	normal datagram.

----------------------------

Test 4100 truncated loose source and record route option

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a remote Driver command request for the IUT

to send a datagram with an 8-byte option field.  This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.  The first two bytes of the

invalid option field that the IUT is requested to send are

no operation options (to permit the truncation of the loose

source and record route option).  The IUT is required to

inform its Remote Driver that the request is invalid.  The

Remote Driver should then request the IUT to send an IP

Remote Driver Specification error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4100 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4101 loose source and record route option length bumped by

one

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an 8-byte option field.  This

datagram should be passed by the IUT to its Remote Driver

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.  The first byte of the

invalid option field that the IUT is requested to send is a

no operation option.  This is followed by a loose source and

record route option whose option length is one more than it

should be.  The IUT is required to inform its Remote Driver

that the request is invalid.  The Remote Driver should then

request the IUT to send an IP Remote Driver Specification

error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4101 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4102 loose source and record route option one byte too long

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an 8-byte option field.  This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.  The option field contains a

loose source and record route option whose length is one

more byte than it should be.  The IUT is required to inform

its Remote Driver that the request is invalid.  The Remote

Driver should then request the IUT to send an IP Remote

Driver Specification error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4102 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4103 loose source and record route option one byte too short

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a loose source and

record route option whose length is one less byte than it

should be.  The IUT is required to inform its Remote Driver

that the request is invalid.  The Remote Driver should then

request the IUT to send an IP Remote Driver Specification

error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "Ident = 4103 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4104 loose source and record route option length of 0

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a loose source and

record route option whose option length is set to 0.  The IUT

is required to inform its Remote Driver that the request is

invalid.  The Remote Driver should then request the IUT to

send an IP Remote Driver Specification error message to the

IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4104 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram

Failure:  Any response other than the display of the error

message datagram.

Test 4105 loose source and record route option length of 377

octal

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a loose source and

record route option whose option length is set to 377 octal.

The IUT is required to inform its Remote Driver that the

request is invalid.  The Remote Driver should then request

the IUT to send an IP Remote Driver Specification error

message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "indent = 4105 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4106 loose source and record route option length of 353

octal

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a loose source and

record route option whose option length is set to 353 octal.

The IUT is required to inform its Remote Driver that the

request is invalid.  The Remote Driver should then request

the IUT to send an IP Remote Driver Specification error

message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4106 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4107 loose source and record route option length of 177

octal

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a loose source and

record route option whose option length is set to 177 octal.

The IUT is required to inform its Remote Driver that the

request is invalid.  The Remote Driver should then request

the IUT to send an IP Remote Driver Specification error

message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4107 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4108 loose source and record route option length of 203

octal

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a loose source and

record route option whose option length is set to 203 octal.

The IUT is required to inform its Remote Driver that the

request is invalid.  The Remote Driver should then request

the IUT to send an IP Remote Driver Specification error

message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4108 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4109 loose source and record route option length of 207

octal

-Can the IUT detect an option that is't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a loose source and

record route option whose option length is set to 207 octal.

The IUT is required to inform its Remote Driver that the

request is invalid.  The Remote Driver should then request

The IUT to send an IP Remote Driver Specification error

message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4109 error" in its 18-

byte data field.  The IP Tester should also display "IHL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4110 loose source and record route option pointer bumped by

1

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a loose source and

the record route option whose option pointer is one more than it

should be.  The IUT is required to inform its Remote Driver

that the request is invalid.  the Remote Driver should then

request the IUT to send an IP Remote Driver Specification

error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4110 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4111 loose source and record route option pointer one short

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a loose source and

record route option whose option pointer is one less than it

should be.  The IUT is required to inform its Remote Driver

that the request is invalid.  The Remote Driver should then

request the IUT to send an IP Remote Driver Specificiation

error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "indent = 4111 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4112 loose source and record route option pointer of 300

octal

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a loose source and

record route option whose option pointer is set to 300

octal.  The IUT is required to inform its Remote Driver that

the request is invalid.  The Remote Driver should then

request the IUT to send an IP Remote Driver Specification

error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4112 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram

Failure:  Any response other than the display of the error

message datagram.

Test 4113 loose source and record route option pointer of 174

octal

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  the option field contains a loose source and

record route option whose option pointer is set to 174

octal.  The IUT is required to inform its Remote Driver that

the request is invalid.  The Remote Driver should then

request the IUT to send an IP Remote Driver Specification

error message to the IP Tester.

-Verification:  Thge IP Tester will display the error message

datagram that must contain "ident = 4113 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4114 loose source and the record route with bad option length

and option pointer

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a loose source and

record route option whose option length and option pointer

have both been bumped by 12.  The IUT is required to inform

its Remote Driver that the request is invalid.  The Remote

Driver should then request the IUT to send an IP Remote

Driver Specification error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4114 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error messaage datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4115 loose source and record route option with bad option

pointer

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Dirver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a single address

loose source and record route option whose option pointer is

set to just its maximum (at any IP) legal value.  The IUT is

required to inform its Remote Driver that the request is

invalid.  The Remote Driver should then request the IUT to

send an IP Remote Driver Specification error message to the

IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4115 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4116 loose source and record route option with bad option

pointer

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a two address loose

source and record route option whose option pointer is set

to just its maximum (at any IP) legal value.  The IUT is

required to inform its Remote Driver that the request is

invalid.  The Remote Driver should then request the IUT to

send an IP Remote Driver Specification error message to the

IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4116 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4117 loose source and record route option duplicated

-Can the IUT detect an option field that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains two identical loose

source and record route options.  The IUT is required to

inform its Remote Driver that the request is invalid.  The

Remote Driver should then request the IUT to send an IP

Remote Driver Specification error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4117 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the errror message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4118 two loose source and record route options

-Can the IUT detect an option field that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains two loose source

and record route options.  The IUT is required to inform its

Remote Driver that the request is invalid.  The Remote Driver

should then request the IUT to send an IP Remote Driver

Specification error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4118 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4119 ordinary datagram after bad LSRR Tests

-Can the IUT send datagram after processing nasty requests?

-Action:  Send a datagram to the IP Implementation Under

Test.  This datagram should be passed by the IUT to its

Remote Driver.  The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

-Verification:  The IP Tester will display the reply datagram

that must contain "ident = 4119" in its 13-byte data field.

Success:  Display of reply datagram requested to be sent.

Failure:  Any response other than the display of the

requested reply datagram.

Test 4120 truncated strict source and record route option

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an 8-byte option field.  This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.  The first two bytes of the

invalid option field that the IUT is requested to send are

no operation options (to permit the truncation of the strict

source and record route option).  The IUT is required to

inform its Remote Driver that the request is invalid.  The

Remote Driver should then request the IUT to send an IP

Remote Driver Specification error message to the IP Tester.

-Verificatioon:  The IP Tester will display the error message

datagram that must contain "ident = 4120 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4121 strict source and record route option length bumped by

1

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an 8-byte option field.  This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.  The first byte of the

invalid option field that the IUT is requested to send is a

no operation option.  This is followed by a strict source and

record route option whose option length is one more than it

should be.  The IUT is required to inform its Remote Driver

that the request is invalid.  The Remote Driver should then

request the IUT to send an IP Remote Driver Specification

error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4121 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4122 strict source and record route option one byte too long

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an 8-byte option field.  This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.  The option field contains a

strict source and record route option whose length is one

more byte than it should be.  The IUT is required to inform

its Remote Driver that the request is invalid.  The Remote

Driver should then request the IUT to send an IP Remote

Driver Specification error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4122 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4123 strict source and record route option one byte too

short

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a strict source and

record route option whose length is one less byte than it

should be.  The IUT is required to inform its Remote Driver

that the request is invalid.  The Remote Driver should then

request the IUT to send an IP Remote Driver Specification

error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4123 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4124 strict source and record route option length of 0

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the Ip Tester.  The option field contains a strict source and

record route option whose option length is set to 0.  The IUT

is required to inform its Remote Driver that the request is

invalid.  The Remote Driver should then request the IUT to

send an IP Remote Driver Specification error message to the

IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4124 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4125 strict source and record route option length of 377

octal

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a strict source and

record route option whose option length is set to 377 octal.

The IUT is required to inform its Remote Driver that the

request is invalid.  The Remote Driver should then request

the IUT to send an IP Remote Driver Specification error

message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4125 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4126 strict source and record route option length of 353

octal

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a strict source and

record route option whose option length is set to 353 octal.

The IUT is required to inform its Remote Driver that the

request is invalid.  The Remote Driver should then request

the IUT to send an IP Remote Driver Specification error

message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4126 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4l27 strict source and record route option length of l77

octal

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a strict source and

record route option whose option length is set to 177 octal.

The IUT is required to inform its Remote Driver that the

request is invalid.  The Remote Driver should then request

the IUT to send an IP Remote Driver Specification error

message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4127 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4128 strict source and record route option length of 203

octal

_Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a strict source and

record route option whose option length is set to 203 octal.

The IUT is required to inform its Remote Driver that the

request is invalid.  The Remote Driver should then request

the IUT to send an IP Remote Driver Specification error

message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4128 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any reponse other than the display of the error

message datagram.

Test 4129 strict source and record route option length of 207

octal

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a strict source and

record route option whose option length is set to 207 octal.

The IUT is required to inform its Remote Driver that the

request is invalid.  The Remote Driver should then request

the IUT to send an IP Remote Driver Specification error

message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4129 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4130 strict source and record route option pointer bumped by

1

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a strict source and

record route option whose option pointer is one more than it

should be.  The IUT is required to inform its Remote Driver

that the request is invalid.  The Remote Driver should then

request the IUT to send an IP Remote Driver Specification

error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4130 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4131 strict source and record route option pointer one short

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a strict source and

record route option whose option pointer is one less than it

should be.  The IUT is required to inform its Remote Driver

that the request is invalid.  The Remote Driver should then

request the IUT to send an IP Remote Driver Specification

error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4131 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4132 strict source and record route option pointer of 300

octal

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a strict source and

record route option whose option pointer is set to 300

octal.  The IUT is required to inform its Remote Driver that

the request is invalid.  The Remote Driver  should then

request the IUT to send an IP Remote Driver Specification

error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4132 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4133 strict source and record route option pointer of 174

octal

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a strict source and

record route option whose option pointer is set to 174

octal.  The IUT is required to inform its Remote Driver that

the request is invalid.  The Remote Driver should then

request the IUT to send an IP Remote Driver Specification

Error Message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4133 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4134 strict source and record route with bad option length

and option pointer

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a strict source and

record route option whose option length and option pointer

have both been bumped by 12.  The IUT is required to inform

its Remote Driver that the request is invalid.  The Remote

Driver should then request the IUT to send an IP Remote

Driver Specification error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4134 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4135 strict source and record route option with bad option

pointer

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a single address

strict source and record route option whose option pointer

is set to just its maximum (at any IP) legal value.  The IUT

is required to inform its Remote Driver that the request is

invalid.  The Remote Driver should then request the IUT to

send an IP Remote Driver Specification error message to the

IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4135 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4136 strict source and record route option with bad option

pointer

-Can the IUT detect an option that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a two address

strict source and record route option whose option pointer

is set to just its maximum (at any IP) legal value.  The IUT

is required to inform its Remote Driver that the request is

invalid.  The Remote Driver should then request the IUT to

send an IP Remote Driver Specification error message to the

IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4136 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4137 strict source and record route option duplicated

-Can the IUT detect an option field that isn't well-formed?

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains two identical

strict source and record route options.  The IUT is required

to inform its Remote Driver that the request is invalid.  The

Remote Driver should then request the IUT to send an IP

Remote Driver specification error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4137 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4138 two strict source and record route options

-Can the IUT detect an option field that isn't well-formed?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains two strict source

and record route options.  The IUT is required to inform its

Remote Driver that the request is invalid.  The Remote Driver

should then request the IUT to send an IP Remote Driver

Specification error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4138 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4139 ordinary datagram after bad SSRR Tests

-Can the IUT send datagram after processing nasty requests?

-Action:  Send a datagram to the IP Implementation Under

Test.  This datagram should be passed by the IUT to its

Remote Driver.  The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

-Verification:  The IP Tester will display the reply datagram

that must contain "ident = 4139" in its 13-byte data field.

Success:  Display of reply datagram requested to be sent.

Failure:  Any response other than the display of the

requested reply datagram.

Test 4140 bad loose source and record route and good stream ID

option

-Can the IUT remember detecting a bad option?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains a bad loose source

and record route option followed by a good stream

identification option.  The IUT is required to inform its

Remote Driver that the request is invalid.  The Remote Driver

should then request the IUT to send an IP Remote Driver

Specification error message to the IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4140 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failuree:  Any response other than the display of the error

message datagram.

Test 4141 loose source and record route and two good security

options

-Can the IUT remember detecting a bad option?

-Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with an option field.  This datagram

should be passed by the IUT to its Remote Driver.  The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.  The option field contains an unclassified

security option followed by a bad loose source and record

route option followed by a top secret security option.  The

IUT is required to inform its Remote Driver that the request

is invalid.  The Remote Driver should then request the IUT to

send an IP Remote Driver Specification error message to the

IP Tester.

-Verification:  The IP Tester will display the error message

datagram that must contain "ident = 4141 error" in its 18-

byte data field.  The IP Tester should also display "HL5"

(internet header length is 5 four-octet units long) to

indicate that the error message datagram does not contain

the invalid option field in its IP header.

Success:  Display of error message datagram.

Failure:  Any response other than the display of the error

message datagram.

Test 4142 ordinary datagram after bad SSRR Tests

-Can the IUT send datagram after processing nasty requests?

-Action:  Send a datagram to the IP Implementation Under

Test.  This datagram should be passed by the IUT to its

Remote Driver.  The IUT's Reote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

-Verification:  The IP Tester will display the reply datagram

that must contain "ident = 4142" in its 13-byte data field.

Success:  Display of reply datagram requested to be sent.

Failure:  Any response other than the display of the

requested reply datagram.

Scenario File OpGlDstBotTest:  "One Gateway Destination" datagram

1300	loose source and record route

1308	strict source and record route

1313	loose source and record route +

strict source and record route +

record route + prespecified timestamp

1314	prespecified timestamp + record route +

strict source and record route +

loose source and record route

1322	fragmented datagram containing record route option

---------------------------------

Test 1300 loose source and record route

- Can the IUT receive a datagram loose routed to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a loose source and record route option

indicating that all the required gateways have been

traversed.  This datagram should be passed by the IUT to its

Remote Driver.  The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification:  The IP Tester will display the reply

datagram that must contain "ident = 1300 " in its 1300 " in its 13-byte

data field.

Success:  Display of reply datagram requested to be sent.

Failure:  Any response other than the display of the

requested reply datagram.

Test 1308 strict source and record route

- Can the IUT receive a datagram strictly routed to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a strict source and record route option

indicating that all the required gateways have been

traversed.  This datagram should be passed by the IUT to its

Remote Driver.  The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification:  The IP Tester will display the reply

datagram that must contain "ident = 1308 " in its 13 byte

data field.

Success:  Display of reply datagram requested to be sent.

Failure:  Any response other than the display of the

requested reply datagram.

Test 1308 strict source and record route

- Can the IUT receive a datagram strictly routed to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a strict source and record route option

indicating that all the required gateways have been

traversed.  This datagram should be passed by the IUT to its

Remote Driver.  The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification:  The IP Tester will display the reply

datagram that must contain "ident = 1308 " in its 13-byte

data field.

Success:  Display of reply datagram requested to be sent.

Failure:  Any response other than the display of the

requested reply datagram.

Test 1313 loose source and record route +

strict source and record route +

record route + prespecified timestamp

- Can the IUT receive a datagram loosely and strictly routed

to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a loose source and record route option, a

strict source and record route option, a record route

option, and a prespecified timestamp option.  Each source

routing option indicates that all the required gateways

have been traversed.  This datagram should be passed by the

IUT to its Remote Driver.  The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

- Verification:  The IP Tester will display the reply

datagram that must contain "ident = 1313 " in its 13-byte

data field.

Success:  Display of reply datagram requested to be sent.

Failure:  Any response other than the display of the

requested reply datagram.

Test 1314 prespecified timestamp + record route +

strict source and record route +

loose source and record route

- Can the IUT receive a datagram strictly and loosely routed

to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a prespecified timestamp option, a record

route option, a strict source and record route option, and

a loose source and record route option.  Each source routing

option indicates that all the required gateways have been

traversed.  This datagram should be passed by the IUT to its

Remote Driver.  The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification:  The IP Tester will display the reply

datagram that must contain "ident = 1314 " in its 13-byte

data field.

Success:  Display of reply datagram requested to be sent.

Failure:  Any response other than the display of the

requested reply datagram.

Test 1322 fragmented datagram containing record route option

- Can the IUT reassemble fragmented datagram containing a

record route option?

- Action:  Send a datagram containing a record route option

(among others) fragmented into two fragments to the IP

Implementation Under Test.  These fragments should be

reassembled and the result passed by the IUT to its Remote

Driver.  The IUT's Remote Driver should request the IUT to

send the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification:  The IP Tester will display the reply

datagram that must contain "ident = 1322 " in its 13-byte

data field.

Success:  Display of reply datagram requested to be sent.

Failure:  Any response other than the display of the

requested reply datagram.

Scenario File OpGlSrcBotTest:  "One Gateway Source" datagrams

CONFIGURATION NOTES:  This scenario requires a different

configuration than that normally used because it is used for

gateway testing.  DCA must provide another network directly

connected to the IUT besides the EDN so the gateway IUT lies

between two networks as required by MIL-STD-1777 section 3.l.j.

An IP host implementation and IP Remote Driver must be

installed in the other network.  The IP Remote Dirver Specification

requires the implementor to provide both of these items as well as

the gateway.

NOTICE:  MIL-STD-1777 states that it "specifies a host IP."  It

is not clear that a gateway can or must conform to the standard.

1100	loose source and record route

1108	strict source and record route

1113	loose source and record route +

strict source and record route +

record route + timestamp and address timestamp

1114	timestamp and address timestamp + record route +

strict source and record route +

loose source and record route

Test 1100 loose source and record route

- Can the IUT forward a loosely routed datagram?

- Action:  Send a datagram to the host IP via the gateway IUT.

This should be passed to the host IP's Remote Driver.  The

Remote Driver and conformant host IP should then arrange to

send a reply datagram to the gateway IUT containing a loose

source and record route option indicating that the datagram

should be forwarded to the IP Tester.  This datagram should be

- Verification:  The IP Tester will display the reply

datagram that must contain "ident = 1100 " in its data

field.  In addition, the display should contain "LOOSE

len=011 ptr=008 00701600002 00702600002" to indicate that

the option was properly processed.

Success:  Display of forwarded reply datagram.

Failure:  Any response other than the display of the

forwarded reply datagram.

Test 1108 strict source and record route

- Can the IUT forward a strictly routed datagram?

- Action:  Send a datagram to the host IP via the gateway IUT.

This should be passed to the host IP's Remote Driver.  The

Remote Driver and conformant host IP should then arrange to

send a reply datagram to the gateway IUT containing a stict

source and record route option indicating that the datagram

should be forwarded to the IP Tester.  This datagram should be

forwarded by the gateway IUT to the IP Tester.

- Verification:  The IP Tester will display the reply

datagram that must contain "ident = 1108 " in its data

field.  In addition, the display should contain "STRICT

len=011 ptr=008 00701600002 00702600002" to indicate that

the option was properly processed.

Success:  Display of forwarded reply datagram.

Failure:  Any response other than the display of the

forwarded reply datagram.

Test 1113 loose source and record route +

strict source and record source +

Record route + timestamp and address timestamp

- Can the IUT forward a loosely and strictly routed diagram?

- Action:  Send a datagram to the host IP via the gateway IUT.

This should be passed to the host IP's Remote Driver.  The

Remote Driver and conformant host IP should then arrange to

send a reply datagram to the gateway IUT containing a loose

source and record route option, and a timestamp and

address timestamp option.  Each source routing option indicates

that the datagram should be forwarded to the IP Tester.  This

datagram should be forwarded by the gateway IUT to the IP

Tester.

- Verification:  The IP Tester will display the reply

datagram that must contain "ident = 1113 " in its data

field.  In addition, the display should contain "LOOSE

len=011 ptr=008 00701600002 00702600002", "STRICT len-011

ptr=008 00701600002 00702600002", "ROUTE len=007 ptr=008

00701600002", and "TIMESTAMP len=012 ptr=013 ovr=00 ALLGT

00701600002" to indicate that the options were properly

processed.  The line containing "TIMESTAMP" should contain a

non-zero value after "00701600002" to indicate that the

option was timestamped.

Success:  Display of forwarded reply datagram.

Failure:  Any response other than the display of the

forwarded reply datagram.

Test 1114 timestamp and address timestamp + record route +

strict source and record route +

loose source and record route

- Can the IUT forward a strictly and loosely routed datagram?

- Action:  Send a datagram to the host IP via the gateway IUT.

This should be passed to the host IP's Remote Driver.  The

Remote Driver and conformant host IP should then arrange to

send a reply datagram to the gateway IUT containing a

timestamp and address timestamp option, a record route option,

a strict source and record route option, and a loose source

and record route option.  Each source routing option indicates

that the datagram should be forwarded to the IP Tester.  This

datagram should be forwarded by the gateway IUT to the IP

Tester.

- Verification:  The IP Tester will display the reply

datagram that must contain "ident = 1114 " in its data

field.  In addition, the display should contain "LOOSE

len=011 ptr=008 00701600002 00702600002", "STRICT len=011

ptr=008 00701600002 00702600002", "ROUTE len=007 ptr=008

00701600002", and "TIMESTAMP len-012 ptr=013 ovr=00 ALLGT

00701600002", to indicate that the options were properly

processed.  The line containing "TIMESTAMP" should contain a

non-zero value after "00701600002" to indicate that the

option was timestamped.

Success:  Display of forwarded reply datagram.

Failure:  Any response other than the display of the

forwarded reply datagram.

Scenario File OpGlSrcTopTest: "One Gateway Source" datagrams

3100	Loose source and record route

3108	strict source and record route

3113	loose source and record route +

strict source and record route +

record route + prespecified timestamp

3114	prespecified timestamp + record route +

strict source and record route +

loose source and record route

3122	record route + no operation +

stream identification +

unclassified security +

end of options list

---------------------------------

Test 3100 loose source and record route

- Can the IUT send a loosely routed datagram?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a loose source and record route

option.  This datagram should be passed by the IUT to its

Remote Driver.  The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification:  The IP Tester will display the reply

datagram that must contain "ident = 3100 " in its data

field.  In addition, the display should contain "LOOSE

len=011 ptr=004 00702600002 03200200001" to indicate that

the option was not improperly processed.

Success:  Display of reply datagram requested to be sent.

Failure:  Any response other than the display of the

requested reply datagram.

Test 3108 strict source and record route

- Can the IUT send a strictly routed datagram?

- Action:   Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a strict source and record route

option. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3108 " in its data

field. In addition, the display should contain "STRICT

len=011 ptr=004 00702600002 03200200001" to indicate that

the option was not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 3113 loose source and record route +

strict source and record route +

record route + prespecified timestamp

- Can the IUT send a loosely and strictly routed datagram?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a loose source and record route

option, a strict source and record route option, a record

route option, and a prespecified timestamp option. This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3113 " in its data

field. In addition, the display should contain "LOOSE

len=011 ptr=004 00702600002 03200200001", "STRICT len=011

ptr=004 00702600002 03200200001", "ROUTE len=007 ptr=004

00000000000", and "TIMESTAMP len=012 ptr=005 ovr=00 MATCH

00702600002 00000000000" to indicate that the options were

not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested-reply datagram.

Test 3114 prespecified timestamp + record route +

strict source and record route +

loose source and record route

- Can the IUT send a strictly and loosely routed datagram?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a prespecified timestamp option, a

record route option, a strict source and record route

option, and a loose source and record route option. This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3114 " in its data

field.  In addition, the display should contain "LOOSE

len=011 ptr=004 00702600002 03200200001", "STRICT len=011

ptr=004 00702600002 03200200001", "ROUTE len=007 ptr=004

00000000000", and "TIMESTAMP len=012 ptr=005 ovr=00 MATCH

00702600002 00000000000" to indicate that the options were

not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 3122 record route + no operation +

stream identification +

unclassified security +

end of options list

- Can the IUT send a datagram with many different options?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a record route option, a no

operation option, a stream identification option, an

unclassified security option, and an end of options list

option. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3122 " in its data

field. In addition, the display should contain "SEC UNCL

comp=00000 hand=hh tcc=tcc", "ROUTE len=007 ptr=004

00000000000", "STREAM 21589" to indicate that the options

were not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Scenario File OpG2DstBotTest: "Two Gateway Destination" datagrams

1301      loose source and record route

1309      strict source and record route

1311      fragmented datagram with strict source and record route

1315      loose source and record route +

record route +

strict source and record route

1316      strict source and record route +

record route +

loose source and record route

1317      loose source and record route +

timestamp only timestamp +

strict source and record route

1318      strict source and record route +

timestamp only timestamp +

loose source and record route

1319      loose source and record route +

prespecified timestamp +

strict source and record route

1320      strict source and record route +

prespecified timestamp +

loose source and record route

1321      fragmented datagram with

strict source and record route +

prespecified timestamp +

loose source and record route

1323      out-of-order fragmented datagram with

strict source and record route +

prespecified timestamp +

loose source and record route

Test 1301 loose source and record route

- Can the IUT receive a datagram loosely routed to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a loose source and record route option

indicating that all the required gateways have been

traversed. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1301 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1309 strict source and record route

- Can the IUT receive a datagram strictly routed to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a strict source and record route option

indicating that all the required gateways have been

traversed. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1309 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1311 fragmented datagram containing a strict source and

record route option

- Can the IUT reassemble fragmented datagram containing a

strict source and record route option?

- Action:  Send a datagram containing a strict source and

record route option fragmented into two fragments to the IP

Implementation Under Test. These fragments should be

reassembled and the result passed by the IUT to its Remote

Driver. The IUT's Remote Driver should request the IUT to

send the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1311 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1315 loose source and record route +

record route +

strict source and record route

- Can the IUT receive a datagram loosely and strictly routed

to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a loose source and record route option, a

record route option, and a strict source and record route

option. Each source routing option indicates that all the

required gateways have been traversed. This datagram should

be passed by the IUT to its Remote Driver. The IUT's Remote

Driver should request the IUT to send the reply datagram

required by the IP Remote Driver Specification to the IP

Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1315 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1316 strict source and record route +

record route +

loose source and record route

- Can the IUT receive a datagram strictly and loosely routed

to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a strict source and record route option, a

record route option, and a loose source and record route

option. Each source routing option indicates that all the

required gateways have been traversed. This datagram should

be passed by the IUT to its Remote Driver. The IUT's Remote

Driver should request the IUT to send the reply datagram

required by the IP Remote Driver Specification to the IP

Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1316 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1317 loose source and record route +

timestamp only timestamp +

strict source and record route

- Can the IUT receive a datagram loosely and strictly routed

to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a loose source and record route option, a

timestamp only timestamp option, and a strict source and

record route option. Each source routing option indicates

that all the required gateways have been traversed. This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1317 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1318 strict source and record route +

timestamp only timestamp +

loose source and record route

- Can the IUT receive a datagram strictly and loosely routed

to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a strict source and record route option, a

timestamp only timestamp option, and a loose source and

record route option. Each source routing option indicates

that all the required gateways have been traversed. This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1318 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1319 loose source and record route +

prespecified timestamp +

strict source and record route

- Can the IUT receive a datagram loosely and strictly routed

to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a loose source and record route option, a

prespecified timestamp option, and a strict source and

record route option. Each source routing option indicates

that all the required gateways have been traversed. This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1319 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1320 strict source and record route +

prespecified timestamp +

loose source and record route

- Can the IUT receive a datagram strictly and loosely routed

to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a strict source and record route option, a

prespecified timestamp option, and a loose source and

record route option. Each source routing option indicates

that all the required gateways have been traversed. This

datagram should be passed by the IUT to its Remote Driver.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1320 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1321 strict source and record route +

prespecified timestamp +

loose source and record route

- Can the IUT reassemble a fragmented datagram strictly and

loosely routed to it?

- Action:  Send a fragmented datagram to the IP

Implementation Under Test containing a strict source and

record route option, a prespecified timestamp option, and a

loose source and record route option. Each source routing

option indicates that all the required gateways have been

traversed. The fragments should be reassembled and the

result passed by the IUT to its Remote Driver. The IUT's

Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to

the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1321 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1323 strict source and record route +

prespecified timestamp +

loose source and record route

- Can the IUT reassemble a fragmented datagram strictly and

loosely routed to it with the fragments delivered in

reverse order?

- Action:  Send a fragmented datagram to the IP

Implementation Under Test containing a strict source and

record route option, a prespecified timestamp option, and a

loose source and record route option. Each source routing

option indicates that all the required gateways have been

traversed. The fragments, which are in reverse order, should

be reassembled and the result passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1323 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Scenario File OpG2SrcBotTest:  "Two Gateway Source" datagrams

CONFIGURATION NOTES: This scenario requires a different

configuration than that normally used because it is used for

gateway testing. DCA must provide another network directly

connected to the IUT besides the EDN so the gateway IUT lies

between two networks as required by MIL-STD-1777 section 3.1.j.

An IP host implementation and IP Remote Driver must be

installed in the other network. The IP Remote Driver Specification

requires the implementor to provide both of these items as well as

the gateway.

NOTICE: MIL-STD-1777 states that it "specifies a host IP."

is not clear that a gateway can or must conform to the standard.

1101      loose source and record route to IP Tester

1102      loose source and record route to host IP

ll05      loose source and record route to unreachable gateway

1109      strict source and record route to IP Tester

1111      strict source and record route to host IP

1112      strict source and record route to unreachable gateway

1115      loose source and record route +

record route +

strict source and record route to IP Tester

1116      strict source and record route +

record route +

loose source and record route to IP Tester

1117      loose source and record route +

timestamp only timestamp +

strict source and record route to IP Tester

1118      strict source and record route +

timestamp only timestamp +

loose source and record route to IP Tester

1119      loose source and record route +

prespecified timestamp +

strict source and record route to IP Tester

1120      strict source and record route +

prespecified timestamp +

loose source and record route to IP Tester

Test 1101 loose source and record route to IP Tester

- Can the IUT forward a loosely routed datagram?

- Action:  Send a datagram to the host IP via the gateway

IUT. This should be passed to the host IP's Remote Driver.

The Remote Driver and conformant host IP should then arrange

to send a reply datagram to the gateway IUT containing a

loose source and record route option indicating that the

datagram should be forwarded to the IP Tester. This datagram

should be forwarded by the gateway IUT to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1101 " in its data

field. In addition, the display should contain "LOOSE

len=Ol5 ptr=008 00701600002 00702600002 03200200001" to

indicate that the option was properly processed.

Success: Display of forwarded reply datagram.

Failure: Any response other than the display of the

forwarded reply datagram.

Test 1102 loose source and record route to host IP

- Can the IUT forward a loosely routed datagram?

- Action:  Send a datagram to the gateway IUT containing a

loose source and record route option indicating that the

datagram should be forwarded to the host-IP. This datagram

should be forwarded by the gateway IUT to the host IP. It

should then be passed by the host IP to its Remote Driver.

The host IP's Remote Driver should request the host IP to

send the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1102 " in its data

field.

Success: Display of reply datagram.

Failure: Any response other than the display of the reply

datagram.

Test 1105 loose source and record route to unreachable gateway

- Can the IUT handle a datagram loosely routed to an

unreachable gateway?

- Action:  Send a datagram to the gateway IUT containing a

loose source and record route option indicating that the

datagram should be forwarded to a nonexistent gateway. This

datagram should be discarded by the gateway IUT. The gateway

IUT may send an ICMP unreachable network or source route

failed message to the IP Tester.

- Verification:  If there is a response datagram displayed

by the IP Tester, its display should contain the string

"ICMP DESTINATION UNREACHABLE ID=01105 NETW" or the string

"ICMP DESTINATION UNREACHABLE ID=01105 ROUT". The string

"ident = 1105 " should not appear because this would

indicate that the datagram was sent to the nonexistent

gateway.

Success: An ICMP unreachable network or source route failed

message displayed by the IP Tester. The total lack of an IP

Tester response datagram is also an acceptable response.

Failure: A response datagram whose IP Tester display

contains the string "ident = 1105 ".

Test 1109 strict source and record route to IP Tester

- Can the IUT forward a strictly routed datagram?

- Action:  Send a datagram to the host IP via the gateway

IUT. This should be passed to the host IP's Remote Driver.

The Remote Driver and conformant host IP should then arrange

to send a reply datagram to the gateway IUT containing a

strict source and record route option indicating that the

datagram should be forwarded to the IP Tester. This datagram

should be forwarded by the gateway IUT to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1109 " in its data

field. In addition, the display should contain "STRICT

len=015 ptr=008 00701600002 00702600002 03200200001" to

indicate that the option was properly processed.

Success: Display of forwarded reply datagram.

Failure: Any response other than the display of the

forwarded reply datagram.

Test 1111 strict source and record route to host IP

- Can the IUT forward a strictly routed datagram?

- Action:  Send a datagram to the gateway IUT containing a

strict source and record route option indicating that the

datagram should be forwarded to the host IP. This datagram

should be forwarded by the gateway IUT to the host IP.  It

should then be passed by the host IP to its Remote Driver.

The host IP's Remote Driver should request the host IP to

send the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1111 " in its data

field.

Success: Display of reply datagram.

Failure: Any response other than the display of the reply

datagram.

Test 1112 strict source and record route to unreachable gateway

- Can the IUT handle a datagram strictly routed to an

unreachable gateway?

- Action:  Send a datagram to the gateway IUT containing a

strict source and record route option indicating that the

datagram should be forwarded to a nonexistent gateway. This

datagram should be discarded by the gateway IUT. The gateway

IUT may send an ICMP unreachable network or source route

failed message to the IP Tester.

- Verification:  If there is a response datagram displayed

by the IP Tester, its display should contain the string

"ICMP DESTINATION UNREACHABLE ID=01112 NETW" or the string

"ICMP DESTINATION UNREACHABLE ID=01112 ROUT". The string

"ident = 1112 " should not appear because this would

indicate that the datagram was sent to the nonexistent

gateway.

Success: An ICMP unreachable network or source route failed

message displayed by the IP Tester. The total lack of an IP

Tester response datagram is also an acceptable response.

Failure: A response datagram whose IP Tester display

contains the string "ident = 1112 ".

Test 1115 loose source and record route +

record route +

strict source and record route to IP Tester

- Can the IUT forward a loosely and strictly routed datagram?

- Action:  Send a datagram to the host IP via the gateway

IUT. This should be passed to the host IP's Remote Driver.

The Remote Driver and conformant host IP should then send a

reply datagram to the gateway IUT containing a loose source

and record route option, a record route option, and a strict

source and record route option. The loose source and record

route option indicates that the datagram may be sent on to

its final destination. The strict source and record route

option indicates that it should be sent to the IP Tester

first. This datagram should be forwarded by the gateway IUT

to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1115 " in its data

field. In addition, the display should contain "LOOSE

len=011 ptr=008 00701600002 03200200001", "STRICT len=015

ptr=008 00701600002 00702600002 03200200001", and "ROUTE

len=007 ptr=008 00701600002" to indicate that the options

were properly processed.

Check each value carefully!!

Success: Display of forwarded reply datagram.

Failure: Any response other than the display of the

forwarded reply datagram.

Test 1116 strict source and record route +

record route +

loose source and record route to IP Tester

- Can the IUT forward a strictly and loosely routed datagram?

- Action:  Send a datagram to the host IP via the gateway

IUT. This should be passed to the host IP's Remote Driver.

The Remote Driver and conformant host IP should then send a

reply datagram to the gateway IUT containing a strict source

and record route option, a record route option, and a loose

source and record route option. The loose source and record

route option indicates that the datagram may be sent on to

its final destination. The strict source and record route

option indicates that it should be sent to the IP Tester

first. This datagram should be forwarded by the gateway IUT

to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1116 " in its data

field. In addition, the display should contain "LOOSE

len=011 ptr=008 00701600002 03200200001", "STRICT len=015

ptr=008 00701600002 00702600002 03200200001", and "ROUTE

len=007 ptr=008 00701600002" to indicate that the options

were properly processed.

Success: Display of forwarded reply datagram.

Failure: Any response other than the display of the

forwarded reply datagram.

Test 1117 loose source and record route +

timestamp only timestamp +

strict source and record route to IP Tester

- Can the IUT forward a loosely and strictly routed datagram?

- Action:  Send a datagram to the host IP via the gateway

IUT. This should be passed to the host IP's Remote Driver.

The Remote Driver and conformant host IP should then send a

reply datagram to the gateway IUT containing a loose source

and record route option, a timestamp only timestamp option,

and a strict source and record route option. The loose

source and record route option indicates that the datagram

may be sent on to its final destination. The strict source

and record route option indicates that it should be sent to

the IP Tester first. This datagram should be forwarded by

the gateway IUT to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1117 " in its data

field.  In addition, the display should contain "LOOSE

len=011 ptr=008 00701600002 03200200001", "STRICT len=015

ptr=008 00701600002 00702600002 03200200001", and

"TIMESTAMP len=012 ptr=010 ovr=00 TIMSO " to indicate that

the options were properly processed. The line containing

"TIMESTAMP" should contain a non-zero value after "TIMSO"

to indicate that the option was timestamped.

Check each value carefully!!!

Success: Display of forwarded reply datagram.

Failure: Any response other than the display of the

forwarded reply datagram.

Test 1118 strict source and record route +

timestamp only timestamp +

loose source and record route to IP Tester

- Can the IUT forward a strictly and loosely routed datagram?

- Action:  Send a datagram to the host IP via the gateway

IUT. This should be passed to the host IP's Remote Driver.

The Remote Driver and conformant host IP should then send a

reply datagram to the gateway IUT containing a strict source

and record route option, a timestamp only timestamp option,

and a loose source and record route option. The loose source

and record route option indicates that the datagram may be

sent on to its final destination. The strict source and

record route option indicates that it should be sent to the

IP Tester first. This datagram should be forwarded by the

gateway IUT to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1118 " in its data

field. In addition, the display should contain "LOOSE

len=011 ptr=008 00701600002 03200200001", "STRICT len=Ol5

ptr=008 00701600002 00702600002 03200200001", and

"TIMESTAMP len=012 ptr=010 ovr=00 TIMSO " to indicate that

and record route option, a prespecified timestamp option,

and a loose source and record route option. The loose source

and record route option doesn't include the address of the

gateway IUT, but the strict source and record route option

does. This datagram should be forwarded by the gateway IUT

to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1120 " in its data

field. In addition, the display should contain "LOOSE

len=011 ptr=004 00702600002 03200200001", "STRICT len=0l5

ptr=008 00701600002 00702600002 03200200001", and

"TIMESTAMP len=012 ptr=013 ovr=00 MATCH 00701600002" to

indicate that the options were properly processed. The line

containing "TIMESTAMP" should contain a non-zero value

after "00701600002" to indicate that the option was

timestamped.

Success: Display of forwarded reply datagram.

Failure: Any response other than the display of the

forwarded reply datagram.

Scenario File OpG2SrcTopTest:  "Two Gateway Source" datagrams

3101      loose source and record route to IP Tester

3102      strict source and record route to disjoint gateway

3109      strict source and record route to IP Tester

3115      loose source and record route +

record route +

strict source and record route to IP Tester

3116      strict source and record route +

record route +

loose source and record route to IP Tester

3117      loose source and record route +

timestamp only timestamp +

strict source and record route to IP Tester

3118      strict source and record route +

timestamp only timestamp +

loose source and record route to IP Tester

3119      loose source and record route +

prespecified timestamp +

strict source and record route to IP Tester

3120      strict source and record route +

prespecified timestamp +

loose source and record route to IP Tester

Test 3101 loose source and record route

- Can the IUT send a loosely routed datagram?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a loose source and record route

option. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3101 " in its data

field.  In addition, the display should contain "LOOSE

len=015 ptr=004 00702600002 14300200001 03200200001" to

indicate that the option was not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 3102 strict source and record route to IP Tester

- Can the IUT detect it shouldn't send a defective strictly

routed datagram?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a strict source and record route

option. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification. The IUT is required to inform its

Remote Driver that the request is invalid because the first

gateway is not on the IUT's network. The Remote Driver

should then request the IUT to send an IP Remote Driver

Specification error message to the IP Tester.

- Verification: The IP Tester will display the error

message datagram that must contain "ident = 3101 error" in

its 18-byte data field.

Success: Display of error message datagram.

Failure: Any response other than the display of the error

message datagram.

Test 3109 strict source and record route

- Can the IUT send a strictly routed datagram?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a strict source and record route

option. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3109 " in its data

field. In addition, the display should contain "STRICT

len=015 ptr=004 00702600002 14300200001 03200200001" to

indicate that the option was not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 3115 loose source and record route +

record route +

strict source and record route to IP Tester

- Can the IUT send a loosely and strictly routed datagram?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a loose source and record route

option, a record route option, and a strict source and

record route option. This datagram should be passed by the

IUT to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3115 " in its data

field. In addition, the display should contain "LOOSE

len=011 ptr=004 00702600002 03200200001", "STRICT len=015

ptr=004 00702600002 14300200001 03200200001", "ROUTE

len=007 ptr=004 00000000000" to indicate that the options

were not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 3116 strict source and record route +

record route +

loose source and record route to IP Tester

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3103 " in its data

field. In addition, the display should contain "LOOSE

len=019 ptr=004 00702600002 14300200001 00702600002

03200200001" to indicate that the option was not improperly

processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 3110 strict source and record route

- Can the IUT send a strictly routed datagram?

- Action:  Send a datagram to the IP Implementation Under Test

containing a Remote Driver command request for the IUT to send

a datagram with a strict source and record route option. This

datagram should be passed by the IUT to its Remote Driver. The

IUT's Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to the

IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3110 " in its data

field. In addition, the display should contain "STRICT

len=019 ptr=004 00702600002 14300200001 00702600002

03200200001" to indicate that the option was not improperly

processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Scenario File OpG6DstBotTest: "Six Gateway Destination" datagram

1306      loose source and record route

Test 1306 loose source and record route

- Can the IUT receive a datagram loosely routed to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a loose source and record route option

indicating that all the required gateways have been

traversed. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Can the IUT send a strictly and loosely routed datagram?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a strict source and record route

option, a record route option, and a loose source and record

route option. This datagram should be passed by the IUT to

its Remote Driver. The IUT's Remote Driver should request

the IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3116 " in its data

field.  In addition, the display should contain "LOOSE

len=011 ptr=004 00702600002 03200200001", "STRICT len=015

ptr=004 00702600002 14300200001 03200200001", "ROUTE

len=007 ptr=004 00000000000" to indicate that the options

were not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 3117 loose source and record route +

timestamp only timestamp +

strict source and record route to IP Tester

- Can the IUT send a loosely and strictly routed datagram?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a loose source and record route

option, a timestamp only timestamp option, and a strict

source and record route option. This datagram should be

passed by the IUT to its Remote Driver. The IUT's Remote

Driver should request the IUT to send the reply datagram

required by the IP Remote Driver Specification to the IP

Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3117 " in its data

field.  In addition, the display should contain "LOOSE

len=011 ptr=004 14300200001 03200200001", "STRICT len=015

ptr=004 00702600002 14300200001 03200200001", "TIMESTAMP

len=012 ptr=005 ovr=00 TIMSO 00000000000 00000000000" to

indicate that the options were not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 3118 strict source and record route +

timestamp only timestamp +

loose source and record route to IP Tester

- Can the IUT send a strictly and loosely routed datagram?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a strict source and record route

option, a timestamp only timestamp option, and a loose

source and record route option. This datagram should be

passed by the IUT to its Remote Driver. The IUT's Remote

Driver should request the IUT to send the reply datagram

required by the IP Remote Driver Specification to the IP

Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3118 " in its data

field.  In addition, the display should contain "LOOSE

len=011 ptr=004 14300200001 03200200001", "STRICT len=015

ptr=004 00702600002 14300200001 03200200001", "TIMESTAMP

len=012 ptr=005 ovr=00 TIMSO 00000000000 00000000000" to

indicate that the options were not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 3119 loose source and record route +

prespecified timestamp +

strict source and record route to IP Tester

- Can the IUT send a loosely and strictly routed datagram?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a loose source and record route

option, a prespecified timestamp option, and a strict source

and record route option. This datagram should be passed by

the IUT to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3119 " in its data

field. In addition, the display should contain "LOOSE

len=011 ptr=004 00702600002 03200200001", "STRICT len=015

ptr=004 00702600002 14300200001 03200200001", "TIMESTAMP

len=012 ptr=005 ovr=00 MATCH 00702600002 00000000000" to

indicate that the options were not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 3120 strict source and record route +

prespecified timestamp +

loose source and record route to IP Tester

- Can the IUT send a strictly and loosely routed datagram?

- Action:  Send a datagram to the IP Implementation Under

Test containing a Remote Driver command request for the IUT

to send a datagram with a strict source and record route

option, a prespecified timestamp option, and a loose source

and record route option. This datagram should be passed by

the IUT to its Remote Driver. The IUT's Remote Driver should

request the IUT to send the reply datagram required by the

IP Remote Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3120 " in its data

field. In addition, the display should contain "LOOSE

len=011 ptr=004 00702600002 03200200001", "STRICT len=015

ptr=004 00702600002 14300200001 03200200001", "TIMESTAMP

len=012 ptr=005 ovr=00 MATCH 00702600002 00000000000" to

indicate that the options were not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Scenario File OpG3DstBotTest:  "Three Gateway Destination" datagrams

1303      loose source and record route

1307      fragmented datagram with loose source and record route

1310      strict source and record route

1312      fragmented datagram with strict source and record route

Test 1303 loose source and record route

- Can the IUT receive a datagram loosely routed to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a loose source and record route option

indicating that all the required gateways have been

traversed. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1303 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1307 fragmented datagram containing a loose source and record

route option

- Can the IUT reassemble fragmented datagram containing a

loose source and record route option?

- Action:  Send a datagram containing a loose source and

record route option fragmented into two fragments to the IP

Implementation Under Test. These fragments should be

reassembled and the result passed by the IUT to its Remote

Driver. The IUT's Remote Driver should request the IUT to

send the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1307 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1310 strict source and record route

- Can the IUT receive a datagram strictly routed to it?

- Action:  Send a datagram to the IP Implementation Under

Test containing a strict source and record route option

indicating that all the required gateways have been

traversed. This datagram should be passed by the IUT to its

Remote Driver. The IUT's Remote Driver should request the

IUT to send the reply datagram required by the IP Remote

Driver Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1310 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1312 fragmented datagram containing a strict source and record

route option

- Can the IUT reassemble fragmented datagram containing a

strict source and record route option?

- Action:  Send a datagram containing a strict source and

record route option fragmented into two fragments to the IP

Implementation Under Test. These fragments should be

reassembled and the result passed by the IUT to its Remote

Driver. The IUT's Remote Driver should request the IUT to

send the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1312 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Scenario File OpG3SrcBotTest:  "Three Gateway Source" datagrams

CONFIGURATION NOTES: This scenario requires a different

configuration than that normally used because it is used for

gateway testing. DCA must provide another network directly

connected to the IUT besides the EDN so the gateway IUT lies

between two networks as required by MIL-STD-1777 section 3.1.j.

An IP host implementation and IP Remote Driver must be

installed in the other network. The IP Remote Driver Specification

requires the implementor to provide both of these items as well as

the gateway.

NOTICE: MIL-STD-1777 states that it "specifies a host IP." It

is not clear that a gateway can or must conform to the standard.

1103      loose source and record route to IP Tester

1104      loose source and record route to host IP

1107      strict source and record route to IP Tester

1110      strict source and record route to host IP

Test 1103 loose source and record route to IP Tester

- Can the IUT forward a loosely routed datagram?

- Action:  Send a datagram to the host IP via the gateway

IUT. This should be passed to the host IP's Remote Driver.

The Remote Driver and conformant host IP should then arrange

to send a datagram to the gateway IUT containing a loose

source and record route option indicating that the datagram

should be forwarded to the IP Tester. This datagram should

be forwarded by the gateway IUT to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1103 " in its data

field.  In addition, the display should contain "LOOSE

len=019 ptr=010 00701600002 00702600002 14300200001

03200200001" to indicate that the option was properly

processed.

Success: Display of forwarded reply datagram.

Failure: Any response other than the display of the

forwarded reply datagram.

Test 1104 loose source and record route to host IP

- Can the IUT forward a loosely routed datagram?

- Action:  Send a datagram to the gateway IUT containing a

loose source and record route option indicating that the

datagram should be forwarded to the host IP. This datagram

should be forwarded by the gateway IUT to the host IP. It

should then be passed by the host IP to its Remote Driver.

The host IP's Remote Driver should request the host IP to

send the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1104 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Test 1107 strict source and record route to IP Tester

- Can the IUT forward a strictly routed datagram?

- Action:  Send a datagram to the host IP via the gateway

IUT. This should be passed to the host IP's Remote Driver.

The Remote Driver and conformant host IP should then arrange

to send a datagram to the gateway IUT containing a strict

source and record route option indicating that the datagram

should be forwarded to the IP Tester. This datagram should

be forwarded by the gateway IUT to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1306 in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Scenario File OpG6SrcBotTest:  "Six Gateway Source" datagrams

CONFIGURATION NOTES: This scenario requires a different

configuration than that normally used because it is used for

gateway testing. DCA must provide another network directly

connected to the IUT besides the EDN so the gateway IUT lies

between two networks as required by MIL-STD-1777 section 3.1.j.

An IP host implementation and IP Remote Driver must be

installed in the other network. The IP Remote Driver Specification

requires the implementor to provide both of these items as well as

the gateway.

NOTICE: MIL-STD-1777 states that it "specifies a host IP." It

is not clear that a gateway can or must conform to the standard.

1106      loose source and record route to IP Tester

1121      loose source and record route to host IP

Test 1106 loose source and record route to IP Tester

- Can the IUT forward a loosely routed datagram?

- Action:  Send a datagram to the host IP via the gateway

IUT. This should be passed to the host IP's Remote Driver.

The Remote Driver and conformant host IP should then arrange

to send a datagram to the gateway IUT containing a loose

source and record route option indicating that the datagram

should be forwarded to the IP Tester. This datagram should

be forwarded by the gateway IUT to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1106 " in its data

field. In addition, the display should contain "LOOSE

len=027 ptr=010 00701600002 00702600002 14300200001

00702600002 14300200001 00702600002 03200200001" to

indicate that the option was properly processed.

Success: Display of forwarded reply datagram.

Failure: Any response other than the display of the

forwarded reply datagram.

Test 1121 loose source and record route to host IP

- Can the IUT forward a loosely routed datagram?

- Action:  Send a datagram to the gateway IUT containing a

loose source and record route option indicating that the

datagram should be forwarded to the host IP. This datagram

should be forwarded by the gateway IUT to the host IP.  It

should then be passed by the host IP to its Remote Driver.

The host IP's Remote Driver should request the host IP to

send the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 1121 " in its 13-byte

data field.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Scenario File OpG6SrcTopTest: "Six Gateway Source" datagram

Test 3106 loose source and record route

Test 3106 loose source and record route

- Can the IUT send a loosely routed datagram?

- Action:  Send a datagram to the IP Implementation Under Test

containing a Remote Driver command request for the IUT to send

a datagram with a loose source and record route option. This

datagram should be passed by the IUT to its Remote Driver. The

IUT's. Remote Driver should request the IUT to send the reply

datagram required by the IP Remote Driver Specification to the

IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 3106 " in its data

field.  In addition, the display should contain "LOOSE

len=03l ptr=004 00702600002 14300200001 00702600002

14300200001 00702600002 14300200001 03200200001" to

indicate that the option was not improperly processed.

Success: Display of reply datagram requested to be sent.

Failure: Any response other than the display of the

requested reply datagram.

Scenario File OptBdBotTest:  "Bottom-up Bad Option Field" datagrams

2000      security option without copy bit set

2001      loose source and record route option without copy bit set

2002      timestamp option without bit 6 set

2003      stream ID option without copy bit set

2004      strict source and record route option without copy bit

set

2005      "option" with option type of 77 octal

2006      "option" with option type of 101 octal

2007      "option" with option type of 177 octal

2008      record route option with copy bit set

2009      prespecified timestamp option with copy bit set

2010      end of options list option with copy bit set

2011      no operation option with copy bit set

2012      ordinary datagram after bad option type tests

2013      truncated security option

2014      security option length bumped by 1

2015      security option one byte too long

2016      security option one byte too short

2017      security option length of 0

2018      security option length of 377 octal

2019      security option length of 353 octal

2020      security option length of 177 octal

2021      security option length of 200 octal

2022      security option length of 213 octal

2023      security option duplicated

2024      two security options (unclassified, secret)

2025      two security options (secret, unclassified)

2026      security option with an invalid security level

2027      ordinary datagram after bad security option tests

2028      truncated stream identification option

2029      stream identification option length bumped by 1

2030      stream identification option one byte too long

2031      stream identification option one byte too short

2032      stream identification option length of 0

2033      stream identification option length of 377 octal

2034      stream identification option length of 354 octal

2035      stream identification option length of 177 octal

2036      stream identification option length of 200 octal

2037      stream identification option length of 204 octal

2038      stream identification option duplicated

2039      two different stream identification options

2040      bad security level + good stream identification option

2041      bad security length + good stream identification option

2042      ordinary datagram after bad stream ID option tests

2043      truncated record route option

2044      record route option length bumped by 1

2045      record route option one byte too long

2046      record route option one byte too short

2047      record route option length of 0

2048      record route option length of 377 octal

2049      record route option length of 353 octal

2050      record route option length of 177 octal

2051      record route option length of 200 octal

2052      record route option length of 207 octal

2053      record route option pointer bumped by 1

2054      record route option pointer one short

2055      record route option pointer of 300 octal

2056      record route option pointer of 174 octal

2057      record route option pointer with bad option length and

option pointer

2058      record route option with bad option pointer

2059      larger record route option with bad option pointer

2060      record route option duplicated

2061      two different record route options

2062      ordinary datagram after bad record route option tests

2063      truncated timestamp option

2064      timestamp option length bumped by 1

2065      timestamp option one byte too long

2066      timestamp option one byte too short

2067      timestamp option length of 0

2068      timestamp option length of 374 octal

2069      timestamp option  length of 354 octal

2070      timestamp option length of 174 octal

2071      timestamp option length of 200 octal

2072      timestamp option length of 210 octal

2073      timestamp option pointer bumped by 1

2074      timestamp option pointer one short

2075      timestamp option pointer of 301 octal

2076      timestamp option pointer of 175 octal

2077      timestamp option pointer with bad option length and

option pointer

2078      timestamp option with bad option pointer

2079      larger timestamp option with bad option pointer

2080      timestamp option duplicated

2081      two different timestamp options

2082      timestamp option with bad timestamp option format

2083      truncated timestamp option

2084      timestamp option length bumped by 1

2085      timestamp option one byte too long .      .

2086      timestamp option four bytes too short

2087      timestamp option four bytes too short

2088      timestamp option length of 374 octal

2089      timestamp option length of 354 octal

2090      timestamp option length of 174 octal

2091      timestamp option length of 200 octal

2092      timestamp option length of 214 octal

2093      timestamp option pointer bumped by 4

2094      timestamp option pointer four short

2095      timestamp option pointer of 305 octal

2096      timestamp option pointer of 175 octal

2097      timestamp option pointer with bad option length and

option pointer

2098      timestamp option with bad option pointer

2099      ordinary datagram after bad timestamp option tests

Test 2000 security option without copy bit set

- Can the IUT interpret a bad security option?

- Action:  Send a datagram to the IP Implementation Under Test

containing a security option without the copy bit set. The IUT

may discard the datagram and send an optional ICMP Parameter

Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2000

or "ICMP PARAMETER PROBLEM id=02000".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2000 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2001 loose source and record route option without copy bit set

- Can the IUT interpret a bad loose source and record route

option?

- Action:  Send a datagram to the IP Implementation Under Test

containing a loose source and record route option without the

copy bit set. The IUT may discard the datagram and send an

optional ICMP Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2001

or "ICMP PARAMETER PROBLEM id=02001".

Success: An ICMP Parameter Problem m.essage, a reply datagram

containing "ident = 2001 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2002 timestamp option without bit 6 set

- Can the IUT interpret a bad timestamp option?

- Action:  Send a datagram to the IP Implementation Under Test

containing a timestamp option without bit 6 set. The IUT may

discard the datagram and send an optional ICMP Parameter

Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2002

or "ICMP PARAMETER PROBLEM id=02002".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2002 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2003 stream ID option without copy bit set

- Can the IUT interpret a bad security option? -

- Action:  Send a datagram to the IP Implementation Under Test

containing a stream identification option without the copy bit

set. The IUT may discard the datagram and send an optional

ICMP Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2003

or "ICMP PARAMETER PROBLEM id=02003".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2003 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2004 strict source and record route option without copy bit

set

- Can the IUT interpret a bad strict source and record route

option?

- Action:  Send a datagram to the IP Implementation Under Test

containing a strict source and record route option without the

copy bit set. The IUT may discard the datagram and send an

optional ICMP Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2004

or "ICMP PARAMETER PROBLEM id=02004".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2004"  or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2005 "option" with option type of 77 octal

- Can the IUT interpret a bad option type?

- Action:  Send a datagram to the IP Implementation Under Test

containing an illegal "option" with option type of 77 octal.

The IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a response datagram displayed by

the IP Tester, its display ought to contain either "ident =

2005 " or "ICMP PARAMETER PROBLEM id=02005".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2005", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2006 "option" with option type of 101 octal

- Can the IUT interpret a bad option type?

- Action:  Send a datagram to the IP Implementation Under Test

containing an illegal "option" with option type of 101 octal.

The IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2006 "

or "ICMP PARAMETER PROBLEM id=02006'.

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2006", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2007 "option' with option type of 177 octal

- Can the IUT interpret a bad option type?

- Action:  Send a datagram to the IP Implementation Under Test

containing an illegal "option' with option type of 177 octal.

The IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification -

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2007

or "ICMP PARAMETER PROBLEM id=02007".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2007 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2008 record route option with copy bit set

- Can the IUT interpret a bad record route option?

- Action:  Send a datagram to the IP Implementation Under Test

containing a record route option with the copy bit set. The

IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2008 "

or "ICMP PARAMETER PROBLEM id=02008".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2008 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2009 prespecified timestamp option with copy bit set

- Can the IUT interpret a bad prespecified timestamp option?

- Actio-n:  Send a datagram to the IP Implementation Under Test

containing a prespecified timestamp option with the copy bit

set. The IUT may discard the datagram and send an optional

ICMP Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2009"

or "ICMP PARAMETER PROBLEM id=02009".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2009 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2010 end of options list option with copy bit set

- Can the IUT interpret a bad end of options list option?

- Action:  Send a datagram to the IP Implementation Under Test

containing a end of options list option with the copy bit set.

The IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver SpecificatIon

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2010

or "ICMP PARAMETER PROBLEM id=02010".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2010 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2011 no operation option with copy bit set

- Can the IUT interpret a bad no operation option?

- Action:  Send a datagram to the IP Implementation Under Test

containing a no operation option with the copy bit set. Tne

IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2011

or "ICMP PARAMETER PROBLEM id=02011".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2011 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2012 ordinary datagram after bad option type tests

- Can the IUT send a datagram after processing nastygrams?

- Action:  Send a datagram to the IP Implementation Under

Test. This datagram should be passed by the IUT to its Remote

Driver. The IUT's Remote Driver should request the IUT to send

the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will disp-lay the reply

datagram that must contain "ident = 2012 " in its 13-byte

data field.

Success: A reply datagram containing "ident = 2012 ".

Failure: Any response other than the display of the

requested reply datagram. The actual failure probably

occurred during tests 2000-2011.

Test 2013 truncated security option

- Can the IUT interpret a truncated security option?

- Action:  Send a datagram to the IP Implementation Under

Test with a 12-byte option field. The first two bytes are no

operation options followed by a truncated security option.

The IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2013 "

or "ICMP PARAMETER PROBLEM id=02013".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2013 "  or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2014 security option length bumped by 1

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under Test

with a 12-byte option field. The first byte is a no operation

option. This is followed by an unclassified security option

whose option length has been bumped by 1. The IUT may discard

the datagram and send an optional ICMP Parameter Problem

message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2014 "

or "ICMP PARAMETER PROBLEM id=02014".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2014 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2015 security option one byte too long

- Can the IUT interpret a defective option?

- Action:  Send a datagram to the IP Implementation Under Test

with a 12-byte option field. This consists of an unclassified

security option that is one byte too long. The IUT may discard

the datagram and send an optional ICMP Parameter Problem

message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2015 "

or "ICMP PARAMETER PROBLEM id=02015".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2015 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2016 security option one byte too short

- Can the IUT interpret a defective option?

- Action:  Send a datagram to the IP Implementation Under Test

with a 12-byte option field. This consists of an unclassified

security option that is one byte too short followed by an an

end of option list option and one byte of padding. The IUT may

discard the datagram and send an optional ICMP Parameter

Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2016"

or "ICMP PARAMETER PROBLEM id=02016".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2016 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2017 security option length of 0

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under

Test with a 12-byte option field. This contains a security

option with an option length of 0. The IUT may discard the

datagram and send an optional ICMP Parameter Problem

message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2017 "

or "ICMP PARAMETER PROBLEM id=02017".

Success: An ICMP Parameter Problem message, a reply

datagram containing "ident = 2017 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2018 security option length of 377 octal

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under

Test with an option field. This contains a security option

with an option length of 377 octal. The IUT may discard the

datagram and send an optional ICMP Parameter Problem

message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2018 "

or "ICMP PARAMETER PROBLEM id=02018".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2018 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2019 security option length of 353 octal

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under Test

with an option field. This contains a security option with an

option length of 353 octal. The IUT may discard the datagram

and send an optional ICMP Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2019 "

or "ICMP PARAMETER PROBLEM id=02019".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2019 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2020 security option length of 177 octal

- Can the IUT interpret a defective option length?

Action:  Send a datagram to the IP Implementation Under Test

with an option field. This contains a security option with an

option length of 177 octal. The IUT may discard the datagram

and send an optional ICMP Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester,  its display ought to contain either "ident = 2020 "    .

or "ICMP PARAMETER PROBLEM id=02020".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2020 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2021 security option length of 200 octal

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under Test

with an option field. This contains a security option with an

option length of 200 octal. The IUT may discard the datagram

and send an optional ICMP Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2021 "

or "ICMP PARAMETER PROBLEM id=02021".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2021 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2022 security option length of 213 octal

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under Test

with an option field. This contains a security option with an

option length of 213 octal (sign bit set).   The IUT may

discard the datagram and send an optional ICMP Parameter

Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2022 "'

or "ICMP PARAMETER PROBLEM id=02022".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2022 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2023 security option duplicated

- Can the IUT interpret a duplicated security option?

- Action:  Send a datagram to the IP Implementation Under Test

whose option field contains two identical security options.

The IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2023 "

or "ICMP PARAMETER PROBLEM id=02023".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2023 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2024 two security options

- Can the IUT interpret two options of the same type?

- Action:  Send a datagram to the IP Implementation Under Test

whose option field contains two different security options.

The IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2024 "

or "ICMP PARAMETER PROBLEM id=02024".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2024 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2025 two security options

- Can the IUT interpret two options of the same type?

- Action:  Send a datagram to the IP Implementation Under Test

whose option field contains two different security options.

This test differs from test 2024 in that the order of the

options has been reversed. The IUT may discard the datagram

and send an optional ICMP Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2025 "

or "ICMP PARAMETER PROBLEM id=02025".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2025 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2026 security option with an invalid security level

- Can the IUT interpret a bad security option?

- Action:  Send a datagram to the IP Implementation Under Test

containing a security option with an invalid security level.

The IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2026 "

or "ICMP PARAMETER PROBLEM id=02026".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2026 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2027 ordinary datagram after bad security option tests

- Can the IUT send a datagram after processing nastygrams?

- Action:  Send a datagram to the IP Implementation Under

Test. This datagram should be passed by the IUT to its Remote

Driver. The IUT's Remote Driver should request the IUT to send

the reply datagram required by the IP Remote Driver

Specification to the IP Tester.

- Verification: The IP Tester will display the reply

datagram that must contain "ident = 2027 " in its 13-byte

data field.

Success: A reply datagram containing "ident = 2027 ".

Failure: Any response other than the display of the

requested reply datagram. The actual failure probably

occurred during tests 2013-2026.

Test 2028 truncated stream identification option

- Can the IUT interpret a truncated stream ID option?

- Action:  Send a datagram to the IP Implementation Under Test

with a 4-byte option field. The first byte is a no operation

option followed by a truncated stream identification option.

The IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2028 "

or "ICMP PARAMETER PROBLEM id=02028".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2028 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2029 stream identification option length bumped by 1

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under Test

with a 4-byte option field consisting of a stream ID option

whose option length has been bumped by 1. The IUT may discard

the datagram and send an optional ICMP Parameter Problem

message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2029 "

or "ICMP PARAMETER PROBLEM id=02029".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2029 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2030 stream identification option one byte too long

- Can the IUT interpret a defective option?

- Action:  Send a datagram to the IP Implementation Under Test

with an 8-byte option field. This contains a stream ID option

that is one byte too long. The IUT may discard the datagram

and send an optional ICMP Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2030 "

or "ICMP PARAMETER PROBLEM id=02030".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2030 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2031 stream identification option one byte too short

- Can the IUT interpret a defective option?       .

- Action:  Send a datagram to the IP Implementation Under Test

with a 4-byte option field. This consists of a stream ID

option that is one byte too short followed by an an end of

option list option. The IUT may discard the datagram and send

an optional ICMP Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2031 "

or "ICMP PARAMETER PROBLEM id=02031".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2031 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2032 stream identification option length of 0

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under Test

with a 4-byte option field. This contains a stream ID option

with an option length of 0. The IUT may discard the datagram

and send an optional ICMP Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2032

or "ICMP PARAMETER PROBLEM id=02032".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2032 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2033 stream identification option length of 377 octal

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under Test

with an option field. This contains a stream identification

option with an option length of 377 octal. The IUT may discard

the datagram and send an optional ICMP Parameter Problem

message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2033"

or "ICMP PARAMETER PROBLEM id=02033".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2033 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2034 stream identification option length of 354 octal

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under Test

with an option field. This contains a stream identification

option with an option length of 354 octal. The IUT may discard

the datagram and send an optional ICMP Parameter Problem

message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specificatiom

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2034"

or "ICMP PARAMETER PROBLEM id=02034".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2034 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2035 stream identification option length of 177 octal

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under Test

with an option field. This contains a stream identification

option with an option length of 177 octal. The IUT may discard

the datagram and send an optional ICMP Parameter Problem

message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2035

or "ICMP PARAMETER PROBLEM id=02035".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2035 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2036 stream identification option length of 200 octal

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under Test

with an option field. This contains a stream identification

option with an option length of 200 octal. The IUT may discard

the datagram and send an optional ICMP Parameter Problem

message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2036 "

or "ICMP PARAMETER PROBLEM id=02036".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2036 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2037 stream identification option length of 204 octal

- Can the IUT interpret a defective option length?

- Action:  Send a datagram to the IP Implementation Under Test

with an option field. This contains a stream identification

option with an option length of 204 octal (sign bit set).   The

IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification:  If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2037"

or "ICMP PARAMETER PROBLEM id=02037".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2037 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2038 stream identification option duplicated

- Can the IUT interpret a duplicated stream ID option?

- Action:  Send a datagram to the IP Implementation Under Test

whose option field contains two identical stream ID options.

The IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2038 "

or "ICMP PARAMETER PROBLEM id=02038".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2038 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2039 two stream identification options

- Can the IUT interpret two options of the same type?

- Action:  Send a datagram to the IP Implementation Under Test

whose option field contains two different stream ID options.

The IUT may discard the datagram and send an optional ICMP

Parameter Problem message.

The IUT may pass the datagram to its Remote Driver instead.

The IUT's Remote Driver should request the IUT to send the

reply datagram required by the IP Remote Driver Specification

to the IP Tester.

- Verification: If there is a datagram displayed by the IP

Tester, its display ought to contain either "ident = 2039 "

or "ICMP PARAMETER PROBLEM id=02039".

Success: An ICMP Parameter Problem message, a reply datagram

containing "ident = 2039 ", or nothing.

Failure: An IP Tester display of a datagram other than an

ICMP Parameter Problem message or reply datagram.

Test 2040 bad security option plus good stream identification

option